FRONT-END ENGINEERING DESIGN (FEED)

100 t/d Sulfuric Acid Plant — DCDA / Elemental Sulfur

EQUIPMENT LIST

Mechanical Equipment Register — Design vs Operating Capacity Consistency Check

| Attribute | Value |
| --- | --- |
| Document No. | PRJ-SA100-FEED-EL-010 |
| Revision | A0 — Issued for Review |
| Project | 100 t/d H₂SO₄ Plant, DCDA Route |
| Date | August 2026 |
| Status | FEED — Consistency-Checked Edition |

# SECTION 1 — VESSELS AND COLUMNS

| Tag No. | Equipment Name | Type | Size (m Dia × m H/L) | Design P (barg) | Design T (°C) | MAWP (barg) | Operating P (barg) | Operating T (°C) | Material | Capacity Basis |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK-101A/B | Molten Sulfur Storage Tank (×2) | Fixed roof, heated | Ø12 × 8 H (each; 4,500 m³ total) | 0.50 | 160 | 0.5 | 0.05 | 140 | CS, rubber-lined | 7-day storage @1,363 kg/h = 9,541 t; 2 × 4,500 t > req. |
| T-301 | Air Drying Tower (DT) | Packed column | Ø2.8 × 18 H | 1.0 | 120 | 1.5 | 0.2 | 65 | CS + brick lining | Gas vel. at surge 2.0 m/s vs flooding 2.5 m/s |
| T-501 | Interpass Absorption Tower (IPAT) | Packed column | Ø3.2 × 22 H | 2.0 | 480 | 2.5 | 0.15 | 420 | CS + acid brick | Gas vel. at surge 1.65 m/s vs flood 2.0 m/s |
| T-502 | Final Absorption Tower (FAT) | Packed column | Ø2.8 × 20 H | 2.0 | 450 | 2.5 | 0.12 | 415 | CS + acid brick | Gas vel. at surge 1.5 m/s |
| R-401 | Catalytic Converter (DCDA) | Multi-bed vertical vessel | Ø4.5 × 28 H | 0.5 | 700 | 0.75 | 0.15 | max 621 at surge | CS shell, 316SS internals | Space vel. at surge 600 h⁻¹ vs max 700 h⁻¹ |
| TK-701A/B | Product H₂SO₄ Storage Tank (×2) | Fixed roof, N₂ blanket | Ø12 × 8 H (each; 2,000 t total) | 0.05 | 60 | 0.1 | 0.01 | 35 | CS + rubber lining | 10-day storage @ 4,167 kg/h |
| D-201 | WHB Steam Drum | Horizontal pressure vessel | Ø1.4 × 4.5 L | 50 | 250 | 50 | 40 | 250 | SA-516 Gr.70 | BFW in + steam out; level 40–60% NL |
| V-301 | Instrument Air Receiver | Pressure vessel | Ø1.0 × 3.0 L | 9.0 | 60 | 9.0 | 7.0 | ambient | CS | Buffer for IA system; ≥10 min autonomy |
| V-702 | Product Acid Weigh Vessel (loading) | Weigh-bridge vessel | Ø2.0 × 4.0 H | 2.0 | 50 | 2.5 | 0.5 | 40 | CS + rubber | Batch loading; 30 t per batch capacity |

# SECTION 2 — HEAT EXCHANGERS

| Tag No. | Service | Type | Area (m²) | Shell Side Fluid / T (°C) | Tube Side Fluid / T (°C) | Design P Shell/Tube (barg) | Duty Normal (kW) | Duty Surge (kW) | Rated Duty (kW) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WHB-201 | Waste Heat Boiler | Fire-tube boiler (gas-in-tube) | 350 m² (tube surface) | BFW/Steam 250°C | SO₂ gas 420–1,050°C | 50 / 1.0 | 16,800 | 18,480 | 21,000 |
| E-401 | Interpass HX (Pass 1–2) | Shell & tube (316SS, fixed TH) | 85 m² | SO₂ gas (cold) 400°C | SO₂ gas (hot) 600°C | 1.0 / 1.0 | 1,650 | 1,815 | 2,000 |
| E-402 | Interpass HX (Pass 2–3) | Shell & tube (316SS) | 55 m² | Cold gas 420°C | Hot gas 520°C | 1.0 / 1.0 | 980 | 1,078 | 1,200 |
| E-403 | Interpass HX (Pass 3–IPAT) | Shell & tube (316SS) | 65 m² | Cold gas returning 415°C | Hot gas 480°C | 1.0 / 1.0 | 1,100 | 1,210 | 1,400 |
| E-404 | Interpass HX (Pass 4–FAT) | Shell & tube (316SS) | 40 m² | Cold gas 390°C | Hot gas 430°C | 1.0 / 1.0 | 620 | 682 | 800 |
| E-501 | IPAT Acid Cooler | Shell & tube (Saramet/Hastelloy) | 180 m² | CW 30–45°C | H₂SO₄ 65–95°C | 6.0 / 6.0 | 2,650 | 2,915 | 3,200 |
| E-502 | FAT Acid Cooler | Shell & tube (Saramet/Hastelloy) | 60 m² | CW 30–45°C | H₂SO₄ 65–90°C | 6.0 / 6.0 | 890 | 979 | 1,100 |
| E-301 | DT Acid Cooler | Graphite block exchanger | 25 m² | CW 30–45°C | H₂SO₄ 65–80°C | 4.0 / 4.0 | 320 | 352 | 420 |
| E-701 | Product Acid Cooler | Shell & tube (Hastelloy C-276) | 28 m² | CW 30–40°C | H₂SO₄ 35–65°C | 6.0 / 6.0 | 420 | 462 | 520 |

NOTE: MARGIN pattern — all interpass and acid heat exchangers: All exchangers are in MARGIN at surge (85–91% of rated duty). This is consistent with the 20% over-design factor applied at sizing. No upsizing is required but all exchangers must be ordered with rated duty clearly specified as the 110% surge duty × 1.1 additional vendor margin. Vendors must confirm that fouling allowance (Rf = 0.0002 m²K/W for acid service) is included in the area calculation.

# SECTION 3 — ROTATING EQUIPMENT (PUMPS)

| Tag No. | Service | Type | Normal Flow (m³/h) | Design Flow (130% rated) (m³/h) | Surge Demand (110% plant) (m³/h) | Motor Power (kW) | Power at Surge (kW) | Motor Rated (kW) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P-101A/B | Molten Sulfur Transfer (D+S) | Gear pump, ATEX, heated casing | 0.75 | 0.98 | 0.82 | 7.5 | 5.8 | 7.5 |
| P-201A/B | BFW to WHB (D+S) | Multistage centrifugal | 7.7 | 10.0 | 8.5 | 45 | 38 | 45 |
| P-301A/B | DT Acid Circulating (D+S) | Centrifugal, Saramet impeller | 45 | 58.5 | 50 | 15 | 12 | 15 |
| P-501A/B | IPAT Acid Circulating (D+S) | Centrifugal, Saramet impeller | 120 | 156 | 132 | 55 | 46 | 55 |
| P-502A/B | FAT Acid Circulating (D+S) | Centrifugal, Saramet impeller | 110 | 143 | 121 | 45 | 40 | 45 |
| P-701A/B | Product Transfer/Storage (D+S) | Centrifugal, rubber-lined | 3.0 | 3.9 | 3.3 | 11 | 9 | 11 |
| P-702A/B | Product Dispatch Pump (D+S) | Centrifugal, Saramet | 3.0 | 3.9 | 3.75 | 22 | 21.8 | 22 |
| P-901A/B | Cooling Water Circ. (D+S) | Centrifugal, epoxy-lined | 240 | 312 | 264 | 75 | 62 | 75 |

# SECTION 4 — ROTATING EQUIPMENT (BLOWERS AND COMPRESSORS)

| Tag No. | Service | Type | Normal Flow (Nm³/h) | Surge Demand (110%) (Nm³/h) | Rated Flow (Nm³/h) | Discharge P (barg) | Motor kW (normal) | Motor kW (surge) | Motor Rated kW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| K-301A | Main Air Blower — Duty | Centrifugal single-stage + VFD | 8,873 | 9,760 | 11,000 | 0.5 | 520 | 572 | 630 |
| K-301B | Main Air Blower — Standby | Centrifugal single-stage + VFD | 8,873 | 9,760 | 11,000 | 0.5 | 520 | 572 | 630 |
| K-901 | Instrument Air Compressor | Reciprocating, oil-free, 2-stage | 180 Nm³/h | 200 | 250 | 7.5 | 18 | 20 | 22 |

# SECTION 5 — FIRED AND SPECIAL EQUIPMENT

| Tag No. | Equipment Name | Type / Description | Key Rating | Design T / P | Operating T / P @ Surge |
| --- | --- | --- | --- | --- | --- |
| BU-201 | Sulfur Furnace | Horizontal cylindrical, refractory-lined (high-alumina castable 1,400°C service) | Thermal input: 21,000 kW rated | 1,200°C / 0.3 barg (design) | 1,080°C / −0.018 barg (surge) |
| F-101 | Molten Sulfur Filter | Pressure leaf filter, SS316, traced | Flow: 2,000 kg/h rated | 160°C / 6 barg | 1,499 kg/h / 0.5 barg |
| WESP-501 | Wet Electrostatic Precipitator (tail gas mist elim.) | Vertical, 4-field, HDPE internals | Gas flow: 11,000 Nm³/h rated; mist removal >99.5% | 100°C / 0.05 barg | 9,760 Nm³/h / 0.05 barg (surge) |
| MBU-501A/B | Acid Mist Eliminators (DT/IPAT/FAT exits) | Fibre-bed type (duty+standby) | Flow per bed: 6,000 Nm³/h | ambient / 0.1 barg | 4,880 Nm³/h each (half total) |
| LA-701 | Tanker Loading Arm (×2) | Top-loading, dry-break, with N₂ blanket | 30,000 kg/h rated per arm; weigh-bridge accuracy ±0.1% | atm / atm | 25,000 kg/h max (1 arm operating) |