FRONT-END ENGINEERING DESIGN (FEED)

100 t/d Sulfuric Acid Plant — DCDA / Elemental Sulfur

START-UP / SHUTDOWN / NORMAL OPERATING PROCEDURES

FEED-Stage Operating Procedures — Step Range Consistency Verified

| Attribute | Value |
| --- | --- |
| Document No. | PRJ-SA100-FEED-SOP-012 |
| Revision | A0 — Issued for Review |
| Project | 100 t/d H₂SO₄ Plant, DCDA Route |
| Date | August 2026 |
| Status | FEED — Consistency-Checked Edition |

# PROCEDURE 1 — COLD START-UP (from ambient, total plant shutdown)

## Pre-requisites (all must be confirmed before proceeding):

| Pre-requisite | Confirmed By | Tag / Location | Acceptable Condition |
| --- | --- | --- | --- |
| All maintenance isolations removed; blinds lifted | Shift Supervisor | Field | Confirmed via PTW close-out |
| SIS system powered and healthy — no active trips | CRO | SIS Panel | All SIS OK lights green; trip log clear |
| DCS in AUTO mode with all loops configured | CRO | DCS Workstation | All PID loops in AUTO with design setpoints |
| Sulfur storage tank level > 30% | CRO | LT-101A | LT-101A reading > 30% on DCS |
| WHB steam drum filled to 40–60% with BFW | CRO | LT-201 | LT-201 reading 40–60% |
| IPAT and FAT acid sumps at 40–60%, concentration 98.0–98.5 wt% | CRO | LT-501/502; AT-501/502 | Both level and concentration in range |
| Process air inlet filter clean (ΔP < 10 mbar) | Field Op. | PI-301 (local gauge) | ΔP < 10 mbar |
| All ESD valves in auto / de-energised safe state | CRO | DCS status display | All SDV-E01, E02, E03, E05 position confirmed on DCS |
| Cooling water supply available (30°C, 4.0 barg) | CRO | PT-901; TT-901 | Within spec |
| Instrument air available (≥7.0 barg, dew pt ≤−40°C) | CRO | PT-IA-001; AT-IA-001 | Within spec |
| Fire and gas detection system healthy | CRO | F&G panel | No active F&G alarms |
| Operator SCBA kits confirmed charged and available | Shift Supervisor | Muster station | Confirmed |

▶  PHASE 1: COOLING WATER AND UTILITIES START-UP (0–30 minutes)

Step 1.1   Start cooling water circulation pumps P-901A (duty). Confirm discharge pressure 4.0 ± 0.2 barg.

✓ Verify: CW supply pressure PT-901 = 3.8–4.2 barg; no vibration on pump casing

Step 1.2   Open CW isolation valves to E-301, E-501, E-502, E-701 acid coolers progressively (allow 2 minutes each).

✓ Verify: CW flow established to each cooler — confirm via local flow indicators

Step 1.3   Start BFW pump P-201A (duty). Set LIC-201 to maintain WHB drum level at 50 ± 5%.

✓ Verify: LT-201 reading stable 45–55%; no high-pressure steam generation yet — drum is cold

Step 1.4   Confirm instrument air ring main pressure 7.0 ± 0.3 barg. Confirm N₂ blanket to product tanks active.

✓ Verify: V-301 IA receiver pressure ≥ 7.0 barg; both product tanks show N₂ flow on local rotameters

▶  PHASE 2: ACID CIRCUIT PRIMING (30–90 minutes)

Step 2.1   Start DT acid circulating pump P-301A. Confirm AIC-501 not yet in auto — DT acid concentration monitoring only.

✓ Verify: P-301A running; LT-301 stable 40–60%; no air flow yet

Step 2.2   Start IPAT acid circulating pump P-501A. Confirm AT-501 reading 98.0–98.5 wt%.

✓ Verify: AT-501 = 98.0–98.5 wt%; LT-501 = 40–60%

⚠ CAUTION: If AT-501 < 97.5 wt%, do NOT proceed — top up with 98.5% make-up acid until target is achieved

Step 2.3   Start FAT acid circulating pump P-502A. Confirm AT-502 reading 98.0–98.5 wt%.

✓ Verify: AT-502 = 98.0–98.5 wt%

Step 2.4   Open dilution water FCV-501 and FCV-502 to approximately 20% output (approximately 45 L/h each). Set AIC-501 and AIC-502 to AUTO.

✓ Verify: AIC-501/502 in AUTO; water flow confirmed on FT-501/502

▶  PHASE 3: MAIN AIR BLOWER START AND CATALYST PRE-HEAT (90–180 minutes)

Step 3.1   Check blower K-301A anti-surge valve in OPEN position. Start K-301A at minimum speed (26 Hz / 50% VFD).

✓ Verify: K-301A running; discharge pressure 0.1–0.2 barg; no vibration; air flow FT-201 > 4,000 Nm³/h

Step 3.2   Slowly increase K-301A VFD speed to achieve air flow of 5,000 Nm³/h (approximately 57% speed). Maintain this flow for catalyst pre-heating.

✓ Verify: FT-201 = 4,800–5,200 Nm³/h; AT-301 (moisture) < 0.1 g/Nm³

⛔ WARNING: Do NOT proceed to ignition if AT-301 moisture reading exceeds 0.15 g/Nm³. High moisture entering the catalyst will cause permanent catalyst deactivation. Check DT acid concentration (target 94–96 wt%) and add make-up 98% acid if required.

Step 3.3   If start-up heater available: energise start-up gas heater to warm converter catalyst to 400°C (Pass 1 inlet). This step takes approximately 90 minutes.

✓ Verify: TT-401 (Pass 1 inlet) reading ≥ 400°C (minimum for sulfur ignition); confirm all 4 pass temperatures rising uniformly

⚠ CAUTION: If TT-401 fails to reach 400°C in 180 minutes, check heater operation and contact Process Engineer before proceeding

▶  PHASE 4: SULFUR IGNITION AND INITIAL COMBUSTION (duration 15–30 minutes)

⛔ WARNING: Confirm all 12 start-up permissives P-01 through P-12 are GREEN on DCS before initiating ignition.

Step 4.1   Confirm ESD system in armed state (all SIS logic energised, solenoids energised, trip functions active).

✓ Verify: DCS SIS status: all SIF armed; no bypasses active

Step 4.2   Open SDV-E01 (sulfur block valve) slowly — DCS command. Position feedback confirmed OPEN.

✓ Verify: SDV-E01 open position feedback on DCS

Step 4.3   Set FIC-101 to MANUAL mode. Gradually open FCV-101 to 10% output — targeting approximately 136 kg/h sulfur flow (10% of design).

✓ Verify: FT-101 reading 120–150 kg/h; TT-201 (furnace T) begins rising within 2 minutes

⚠ CAUTION: If furnace temperature does not begin rising within 3 minutes of sulfur admission, close FCV-101 immediately and purge for 10 minutes with air before re-attempting ignition

Step 4.4   Confirm stable combustion: TT-201 rising and AT-201 (SO₂) reading > 8 vol%.

✓ Verify: TT-201 > 700°C and rising; AT-201 > 8 vol%

Step 4.5   Switch FIC-101 to AUTO (cascade from Governor). Set Master Load to 10%.

✓ Verify: DCS Master Load = 10%; FIC-101 and FIC-201 in AUTO cascade

▶  PHASE 5: LOAD RAMP TO DESIGN (2–4 hours)

Step 5.1   Increase Master Load from 10% to 50% at rate of 5% per 10 minutes (maximum ramp rate). Monitor TT-401 through TT-408 continuously.

✓ Verify: All converter pass temperatures trending smoothly; TT-402 (Pass 1 outlet) must not exceed 610°C during ramp

⚠ CAUTION: If TT-402 approaches 605°C during ramp, pause load increase for 15 minutes until temperatures stabilise

Step 5.2   At 50% load: confirm AT-403 (stack SO₂) < 500 mg/Nm³. Continue ramp to 80%.

✓ Verify: AT-403 < 500 mg/Nm³; all absorption tower levels stable; AT-501/502 = 98.0–98.5 wt%

Step 5.3   At 80% load: confirm WHB steam generation established. Set PIC-202 to AUTO for steam export.

✓ Verify: FT-202 (steam flow) > 6,000 kg/h; PT-202 = 40 ± 1 barg; PIC-202 in AUTO

Step 5.4   At 80%: confirm all acid loops stabilised. Transition AIC-501 and AIC-502 from MANUAL trim to full AUTO.

✓ Verify: Both AIC loops in AUTO; concentrations stable ± 0.1 wt%

Step 5.5   Continue ramp to 100% load at same ramp rate. At 100%: declare Normal Operation.

✓ Verify: AT-403 < 300 mg/Nm³; TT-402 ≤ 600°C; AT-501/502 = 98.0–98.5 wt%; PT-202 = 40 barg; All DCS loops in AUTO

⚠ CAUTION: Complete start-up log entry; notify Shift Supervisor; schedule first product sample to laboratory within 30 min of reaching 100% load

# PROCEDURE 2 — NORMAL OPERATING PROCEDURE (continuous steady-state)

## 2.1 Routine Operator Rounds (every 2 hours)

| Round Item | Location | Check | Accept Range | Action if Outside Range |
| --- | --- | --- | --- | --- |
| Sulfur spray gun condition | Furnace front | No sulfur solidification on nozzle; clean atomization | Visual — clean spray | Notify CRO; prepare spare gun; swap if blocked |
| DT acid level and temperature | T-301 local | Level 40–60%; temperature 60–75°C | LT-301: 40–60%; TT-301: 60–75°C | Adjust make-up acid; check cooler CW flow |
| IPAT and FAT acid sump levels | T-501, T-502 | Level 45–55% | LT-501/502: 45–55% | Adjust level control valves LCV-501/502 |
| Converter body temperature | R-401 field | Uniform temperature on each pass (multipoint TT) | As per DCS TIC setpoints ± 15°C | Notify CRO if deviation > 15°C; check bypass valve position |
| WHB drum water level (local sight glass) | WHB-201 | Level at half-glass | 40–60% of sight glass | Cross-check with LT-201 DCS reading; notify CRO if discrepancy |
| Product acid cooler outlet temperature | E-701 | Acid cool to 30–40°C | TT-701: 30–40°C | Check CW flow; inspect TCV-701 position |
| Product tank level | TK-701 | Both tanks: level and level agreement | LT-701 20–80%; within 5% of LT-702 reading | If difference > 5%: potential level transmitter fault; notify CRO; cross-check with manual dip |
| Stack visual inspection | Stack base / observation | No visible acid mist plume; no unusual colour | Clear / pale white vapour only | If dense white plume: check AT-403 (stack SO₂) and WESP operation; notify HSE |

## 2.2 Load Change Procedure (during normal operation)

Any load change (increase or decrease) shall be made by adjusting the DCS Master Load setpoint only. Direct manipulation of FIC-101 or FIC-201 setpoints is NOT permitted during normal operation.

| Action | Maximum Rate | Minimum Stabilisation Time Before Next Change | Monitor |
| --- | --- | --- | --- |
| Load increase | 5% per 10 minutes | 15 minutes after each 5% increment | TT-402 < 610°C; AT-403 < 400 mg/Nm³; AIC-501/502 stable |
| Load decrease | 10% per 10 minutes | 10 minutes after each 10% decrement | AT-403 trend; AIC-501/502 concentration; no alarms |
| Surge to 110% | 5% per 10 minutes from 100% | 30 minutes monitoring at 110% before declaring stable | TT-402 ≤ 610°C critical; CW temperatures; steam export |
| Return from 110% to 100% | 5% per 5 minutes | 10 minutes | Confirm P1 temp drops below 600°C within 5 min of load reduction |

# PROCEDURE 3 — PLANNED SHUTDOWN (4–6 hours)

⛔ WARNING: Planned Shutdown must be authorised by the Shift Supervisor and notified to the Process Engineer and HSE department at least 1 hour before commencement.

▶  PHASE 1: LOAD REDUCTION (0–90 minutes)

Step S.1   Set Master Load to 80%. Confirm stable operation at 80% for 15 minutes.

✓ Verify: All loops stable; AT-403 < 300 mg/Nm³

Step S.2   Set Master Load to 60%. Continue monitoring converter temperatures and stack SO₂.

✓ Verify: TT-402 < 580°C; AT-403 < 400 mg/Nm³

⚠ CAUTION: If AT-403 > 400 mg/Nm³ at 60% load, reduce to 50% and notify HSE

Step S.3   Set Master Load to 50% (minimum stable turndown). Confirm 20 minutes stable operation.

✓ Verify: AT-403 < 480 mg/Nm³; all loops stable

Step S.4   Set Master Load to 0% (FIC-101 demand → 0). FIC-101 closes FCV-101. Confirm SDV-E01 closes automatically on zero flow.

✓ Verify: FT-101 = 0 kg/h; TT-201 begins to fall; AT-201 SO₂ begins to fall

▶  PHASE 2: FURNACE PURGE AND COOLING (90–240 minutes)

Step S.5   Continue K-301A air blower at 50% speed for air purge of furnace and converter. Minimum 30-minute air purge after sulfur cutoff.

✓ Verify: AT-201 (SO₂) < 1 vol% after 30 minutes of purge

⚠ CAUTION: Do NOT stop air blower until SO₂ < 1 vol% — residual SO₂ must be cleared before duct entry

Step S.6   Once TT-201 < 200°C, progressively reduce blower speed to 30% and maintain for natural cooling.

✓ Verify: TT-201 declining; TT-401 (converter) declining below 200°C

Step S.7   Maintain WHB BFW flow (LIC-201 in AUTO) until drum pressure PT-202 < 5 barg. Then close BFW isolation and drain steam circuit under Process Engineer supervision.

✓ Verify: PT-202 < 5 barg before BFW isolation; steam drum vented

⚠ CAUTION: Critical: BFW must remain on until pressure < 5 barg to prevent dry-fire during heat soak

▶  PHASE 3: ACID CIRCUIT DRAIN AND ISOLATION (240–360 minutes)

Step S.8   Transfer excess product acid from IPAT and FAT sumps to product storage tank TK-701. Confirm LT-701 < 80% (product tank).

✓ Verify: Transfer pump flow confirmed; LT-701 < 80%

Step S.9   Stop acid circulating pumps P-501A and P-502A. Close all acid isolation valves. Drain sumps to designated acid sump collection.

✓ Verify: No acid flow in absorption towers; all drain valves confirmed closed

Step S.10   Stop K-301A blower. Isolate air inlet. Complete shutdown log.

✓ Verify: K-301A stopped; all flows at zero; DCS confirms plant in "shutdown" state

NOTE: Post-shutdown: the plant remains a Major Hazard site even when shutdown. All entry to vessels or confined spaces requires a confined space entry permit, gas testing (O₂ > 19.5%; SO₂ < 0.5 ppm), and SCBA standby.

# PROCEDURES — CONSISTENCY SUMMARY (Key Setpoints in Procedures vs Design Docs)

| Procedure Step | Setpoint Referenced | Design Doc Value | Consistent? |
| --- | --- | --- | --- |
| Phase 3: AT-301 moisture check | 0.15 g/Nm³ limit before ignition | BoD §1.3; II AT-301 alarm AAH 0.15 | Yes |
| Phase 4: Permissive check | All 12 permissives per C&E Doc 6 | C&E Section 4 permissives P-01 to P-12 | Yes |
| Phase 5: TT-402 limit during ramp | 610°C operator caution (before 620°C SIS trip) | C&E SIF-05 TSHH 620°C; II TT-402 | Yes — 10°C operator buffer |
| Phase 5: AT-403 normal target | < 300 mg/Nm³ at 100% load | BoD §1.6; Alarm List AAH 500 mg/Nm³ | Yes |
| Normal op: surge to 110% | TT-402 must remain ≤ 610°C | H&MB FAIL: actual surge value is 621°C — exceeds both the 610°C caution and 620°C SIS trip | CONFLICT: procedures cannot safely operate at 110% until Pass 1 temperature is resolved |
| Shutdown S.7: BFW before <5 barg | BFW isolation only when PT-202 < 5 barg | WHB design; SIF-03 LSLL protection | Yes — explicit in procedure |
| Shutdown S.5: SO₂ purge | AT-201 < 1 vol% before air purge complete | Alarm List AAL-201 8.5 vol% (process); 1 vol% is safe purge level | Yes — 1 vol% is conservative purge criterion |