FRONT-END ENGINEERING DESIGN (FEED)

100 t/d Sulfuric Acid Plant — DCDA / Elemental Sulfur

EMERGENCY PROCEDURES

Emergency Response and Recovery — Setpoint & Response Time Consistency Verified

| Attribute | Value |
| --- | --- |
| Document No. | PRJ-SA100-FEED-EP-013 |
| Revision | A0 — Issued for Review |
| Project | 100 t/d H₂SO₄ Plant, DCDA Route |
| Date | August 2026 |
| Status | FEED — Consistency-Checked Edition |

This document contains automated consistency status flags. Parameters marked FAIL require engineering review before progression to Detailed Engineering.

🚨  EMERGENCY: EMERGENCY 1 — AUTOMATIC ESD ACTUATION (any SIF trip)

## E1.1 Immediate Response (0–2 minutes)

E1.1.1  [CRO]  Confirm ESD actuation on DCS: identify which SIF has tripped (SIS trip log on dedicated SIS screen). Announce on plant PA: "ESD actuation — all field personnel muster at [muster point]."

E1.1.2  [CRO]  Confirm that SDV-E01 (sulfur block) is CLOSED (position feedback on DCS). If position feedback shows OPEN, escalate to MANUAL ESD.

Verify: SDV-E01 "Closed" confirmation on DCS position indicator

E1.1.3  [CRO]  Confirm K-301A (air blower) continues running — air purge of furnace is ESSENTIAL post-ESD. Air flow FT-201 > 4,000 Nm³/h must be maintained.

Verify: FT-201 > 4,000 Nm³/h on DCS

E1.1.4  [Shift Supervisor]  Account for all personnel at muster point within 5 minutes of PA announcement.

Verify: All persons signed in to shift accounted for at muster

E1.1.5  [CRO]  Monitor TT-201 (furnace temperature). Confirm trend downward. If temperature rises after ESD, contact Process Engineer immediately — potential ignition source remaining.

Verify: TT-201 trending down within 3 minutes

## E1.2 SIF-Specific Response Checks

| SIF Tripped | Likely Cause | First Response Action Beyond E1.1 | Recovery Condition (before restart) |
| --- | --- | --- | --- |
| SIF-01 (Furnace T HH 1,150°C) | Sulfur gun blockage → rich burn; air flow loss; refractory degradation | Check TT-201 trend: if still rising after ESD, evacuate 50 m radius and contact fire service. If declining: investigate root cause (check FT-201, AT-202 O₂, sulfur gun condition). | TT-201 < 200°C; root cause identified and corrected; HAZOP closeout of trip event; Shift Supervisor sign-off |
| SIF-02 (Furnace P HH 300 mbar) | Air blower surge; outlet duct blockage; stack damper closed unintentionally | CHECK XV-E04 stack damper position: if CLOSED (pre-correction FAIL state), open manually at field — this is urgent. After correction: investigate duct blockage. | Stack damper open; duct clear; furnace pressure < −10 mbar before restart |
| SIF-03 (WHB Drum Level LL 20%) | BFW pump failure; LCV-201 failure closed; large steam demand spike | Start standby BFW pump P-201B immediately. Set LIC-201 to manual — maximum BFW demand. Do not reduce steam demand until drum level recovers to 30%. | LT-201 > 40%; BFW pump confirmed duty/standby operational; LIC-201 3-element control verified |
| SIF-04 (WHB Steam P HH 46 barg) | Steam export trip; PCV-202 failure closed; excessive steam generation | Open steam dump to atmosphere via manual bypass valve (PV-202-bypass) if available. Reduce BFW if steam generation continues rising. DO NOT vent to atmosphere without HSE notification. | PT-202 < 42 barg stable; PCV-202 operation verified |
| SIF-05 (Converter P1 T HH 620°C) | Plant operating at 110% surge load — this is the DESIGN FAIL scenario. Temperature reached trip setpoint. | Confirm ESD actuation; reduce to 0% load; investigate root cause (confirm this is the design deficiency — bed height issue — per H&MB and SRS records). Do NOT attempt restart at 110% load until process design corrected. | Process design corrected per H&MB FAIL resolution; restart only at ≤100% load until redesign is implemented and validated |
| SIF-06 (Stack SO₂ HH 600 mg/Nm³) | Catalyst degradation; absorption tower acid concentration low; IPAT/FAT pump failure | CHECK AT-501/502 acid concentration. If < 97.5 wt%, dilution water fault (check FCV-501/502). If acid OK, investigate catalyst performance (check all converter pass temperatures). | AT-403 < 400 mg/Nm³ stable; root cause identified; CEMS download for regulatory review |
| SIF-07 (Product Tank L HH 90%) | Dispatch pump failure; tanker not available; over-production | Stop all product transfer from absorption to storage (close product transfer valve). Halt dispatching only if dispatch pump is failed — not if tanker is absent (continue dispatching). Contact dispatch coordinator immediately. | LT-701 < 70% before resuming production; dispatch pump confirmed operational |

🚨  EMERGENCY: EMERGENCY 2 — MAJOR SO₂ RELEASE / TOXIC GAS CLOUD

## E2.1 Recognition and Immediate Actions

| Indicator | Source | Trigger Level | Required Action |
| --- | --- | --- | --- |
| AA-901 area monitor HIGH | F&G system (DCS alarm) | 5 ppm SO₂ | CRO announces PA warning: "SO₂ warning — upwind personnel stay clear of [affected area]." Investigate source immediately. |
| AA-901 area monitor HH | F&G system (automatic) | 25 ppm SO₂ | AUTOMATIC ESD actuation + PA EVACUATION alarm. All non-essential personnel evacuate to upwind muster point. Emergency services: DIAL [site emergency number]. |
| Multiple AA-901 HH | F&G | >25 ppm multiple nodes | Indicates large release. Full site evacuation. Contact emergency services, site EHS Manager, and notify relevant Local Authority. |
| Visible acid mist cloud | Visual (any observer) | Any visible | Report to CRO immediately; CRO initiates PA warning; manual ESD if required. |
| Personnel H₂SO₄ exposure | Any | Any skin/eye contact | Flush immediately with copious water for minimum 20 minutes. Call site first-aider. Hospital evaluation for all acid exposures regardless of apparent severity. |

E2.1  [CRO]  On AA-901-HH (25 ppm): manually initiate TOTAL PLANT ESD via Control Room ESD button if automatic ESD has not actuated. Confirm PA system active.

Verify: Plant ESD confirmed; PA evacuation announcement active

E2.2  [Shift Supervisor]  Account for all personnel. Any person exposed to SO₂ > 5 ppm: escort to fresh air, administer O₂ if available, arrange medical assessment.

Verify: All personnel accounted for; no personnel in toxic zone

E2.3  [CRO]  Identify source: check if release is from furnace/converter (SO₂ gas) or from acid system (acid mist/SO₃). This determines the response priority.

Verify: Source identified from F&G node activation pattern on DCS F&G map

E2.4  [CRO]  Confirm K-301A continues running for air purge. Confirm AT-403 (CEMS) recording for regulatory evidence.

Verify: AT-403 recording; CEMS data logger confirmed active

## E2.2 SO₂ Exposure Limits (for field response planning)

| SO₂ Level | Classification | Personnel Action | PPE Required |
| --- | --- | --- | --- |
| 0–0.5 ppm | LTEL (8-hour TWA limit) | Normal work — routine monitoring | None (normal) |
| 0.5–2 ppm | STEL (15-minute limit) | Time-limit exposure; investigate source | Full face respirator minimum |
| 2–5 ppm | Detectable; discomfort | Leave area; report to CRO | SCBA required |
| 5 ppm | AA-901 HIGH alarm trigger | Non-essential personnel withdraw | SCBA required for all entry |
| 25 ppm | AA-901 HH — IDLH threshold | FULL EVACUATION; emergency services | SCBA — entry ONLY with trained team |
| >100 ppm | Immediately dangerous to life | Extreme hazard; NO ENTRY without full SCBA and buddy system | SCBA + full chemical suit |

🚨  EMERGENCY: EMERGENCY 3 — SULFURIC ACID MAJOR SPILL

## E3.1 Classification and Immediate Actions

| Spill Scale | Volume | Classification | Immediate Action | External Notification |
| --- | --- | --- | --- | --- |
| Minor | < 20 litres | Near-miss / minor incident | Contain with dry sand/earth. Do NOT use water on concentrated H₂SO₄ (violent exotherm). PPE: face shield, acid gloves, acid apron. Recover with absorbent material. | Shift Supervisor log only |
| Medium | 20–1,000 litres | Significant incident | Activate spill kit. Ensure bund valve closed. Dilute carefully with water spray (fine mist from distance). Neutralise with Na₂CO₃ or lime slurry ONLY after initial containment. | EHS Manager + regulatory notification within 24 h |
| Major | > 1,000 litres | Emergency — MAJOR RELEASE | ACTIVATE SITE EMERGENCY PLAN. Evacuate downwind area. Contact fire service. Do NOT approach without full chemical protection suit + SCBA. Contain within bund — verify bund isolation valve CLOSED. | Immediate notification to Environment Agency, Local Authority, and HSE within 1 hour |

⛔ DANGER: CONCENTRATED H₂SO₄ REACTS VIOLENTLY WITH WATER. NEVER add water directly to spilled acid. Only use water mist from a safe distance (>3 m). Risk of spattering and steam explosion if water is poured onto concentrated acid.

E3.1  [CRO]  On notification of major acid spill: initiate production shutdown (reduce to 0% load immediately — do not wait for ESD). Confirm SDV-E05 (product block) CLOSED.

Verify: SDV-E05 closed; production load at 0%

E3.2  [Shift Supervisor]  Confirm bund isolation valve for affected tank area is CLOSED. Confirm bund is containing the spill.

Verify: Visual confirmation from safe distance; CCTV if available

E3.3  [Field Operator (SCBA)]  If safe to do so: position spill response foam/neutralisation equipment at bund perimeter (outside bund). Do NOT enter bund without emergency team leader approval.

E3.4  [Shift Supervisor]  Initiate site emergency muster. Notify EHS Manager and site emergency coordinator.

Verify: All personnel at muster; emergency coordinator contacted

🚨  EMERGENCY: EMERGENCY 4 — LOSS OF INSTRUMENT AIR

## E4.1 Immediate Impact and Response

Loss of instrument air (IA pressure < 5 barg) causes all pneumatic control valves to move to their fail-safe positions simultaneously. This is a significant process disturbance. The fail-safe positions defined in the C&E matrix (Doc 6) are:

| Valve | Fail-Safe Position | Process Effect on IA Loss | Operator Response |
| --- | --- | --- | --- |
| FCV-101 (Sulfur feed) | Fail CLOSED | Sulfur feed stops — furnace temperature drops | Monitor TT-201 declining. If IA restored within 5 min, restart sulfur gradually from Governor setpoint. If > 5 min, initiate controlled shutdown. |
| LCV-201 (BFW) | Fail OPEN | Maximum BFW flow to WHB — risk of drum overfill | Set LIC-201 to manual/override. Monitor LT-201. If drum > 80%, close BFW isolation manually. |
| FCV-501/502 (Dilution water) | Fail CLOSED | Dilution water stops — acid concentration rises | Monitor AT-501/502. If concentration > 98.8 wt%, manually adjust dilution water bypass valve. |
| TCV-501 (IPAT CW) | Fail OPEN | Maximum CW to acid cooler — acid cools below target | Non-critical; temperature will recover when IA restored. |
| SDV-E01 (Sulfur block, ESD) | Fail CLOSED (spring) | Sulfur isolated — plant shuts down | Confirm ESD actuation on SIS panel. If ESD trips, follow E1 procedure. |
| SDV-E02 (Air block, ESD) | Fail OPEN | Air continues — safe purge maintained | Correct and desirable fail-safe position. |

E4.1  [CRO]  Check IA supply pressure PT-IA-001. If < 5 barg: confirm IA compressor K-901 status. Start standby compressor if available.

Verify: K-901B running; IA pressure recovering

E4.2  [CRO]  Monitor all valve positions via DCS: confirm fail-safe positions per table above. Note any valve NOT in expected fail-safe position — this indicates a valve actuator fault.

Verify: All fail-safe positions confirmed via DCS position feedback

E4.3  [CRO]  If IA not restored within 10 minutes: initiate controlled shutdown Procedure 3 (Planned Shutdown Phase 1 onwards). Do not attempt to hold plant on manual override beyond 10 minutes without IA.

Verify: Shutdown procedure initiated; Shift Supervisor notified

🚨  EMERGENCY: EMERGENCY 5 — POWER FAILURE (TOTAL ELECTRICAL)

| Time After Failure | System State | Required Action |
| --- | --- | --- |
| 0–10 seconds | UPS activates for DCS, SIS, F&G, emergency lighting. All MV motors (blower, BFW pump) coast to stop. | CRO: Confirm DCS alive on UPS. Confirm SIS alive — all SIFs remain armed. PA announcement: "Power failure — all personnel muster." |
| 10–30 seconds | K-301A air blower coasts to stop — no air purge. Furnace without air and sulfur (FCV-101 fails closed). | Furnace is dark — no ignition, no combustion. Key risk: residual SO₂ in system. Do NOT attempt manual restart of blower until DG is confirmed live. |
| 0–3 minutes | Emergency diesel generator (DG) should energise MV bus. | CRO: Confirm DG has picked up MV bus. Restart K-301A on DG supply immediately for purge airflow. Confirm BFW pump P-201B available on DG. |
| 3–15 minutes | DG running; air purge active. Plant is in safe shutdown state. | Maintain air purge. Monitor WHB drum level. Do NOT restart sulfur until utility power is restored and root cause identified. |
| >60 minutes without power | WHB will lose pressure; acid circuit may begin to cool and risk solidification in low spots. | Process Engineer decision: drain acid circuits if power restoration not expected within 2 hours. Follow Planned Shutdown Steps S.8–S.10. |

# EMERGENCY PROCEDURES — CONSISTENCY SUMMARY

| Emergency | Issue | Status | Action Required Before Procedures Can Be Relied Upon |
| --- | --- | --- | --- |
| E1: Auto ESD | SIF-05 will trip on every 110% surge operation — not a genuine emergency but a design deficiency | FAIL | Resolve Pass 1 temperature (H&MB FAIL). Until resolved, 110% operation is PROHIBITED and this emergency procedure for SIF-05 is a routine event procedure. |
| E1 SIF-02 | Stack damper XV-E04 Fail Closed during ESD blocks venting — manual compensating measure required | FAIL | Correct valve to Fail Open (C&E FAIL). Until corrected, add mandatory manual field step to open damper within 60 s of ESD in this procedure. |
| E2: SO₂ Release | F&G area monitors: threshold setpoints (5 ppm HIGH; 25 ppm HH) confirmed consistent with occupational exposure limits (WEL) and IDLH values | PASS | No action |
| E3: Acid Spill | Bund sizing (110% of tank volume) confirmed per BoD environmental design criteria | PASS | No action |
| E4: IA Failure | All fail-safe positions confirmed consistent with C&E matrix Doc 6 | PASS | No action — verify at FAT testing |
| E5: Power failure | DG transfer time and air blower restart timing needs dynamic simulation validation | MARGIN | Confirm DG auto-start time < 30 s; confirm blower coast-down time to confirm purge interruption duration |
| All procedures | SIS trip setpoints in procedures match SRS Doc 8 and C&E Doc 6 | PASS | All emergency ESD setpoints consistent |