FRONT-END ENGINEERING DESIGN (FEED)

100 t/d Sulfuric Acid Plant — DCDA / Elemental Sulfur

VENDOR DATASHEETS — EQUIPMENT PURCHASE SPECIFICATIONS

FEED-Stage Datasheets for All Major Equipment Items — Specified vs Operating Range Consistency Verified

| Attribute | Value |
| --- | --- |
| Document No. | PRJ-SA100-FEED-DS-014 |
| Revision | A0 — Issued for Review |
| Project | 100 t/d H₂SO₄ Plant, DCDA Route |
| Date | August 2026 |
| Status | FEED — Consistency-Checked Edition |

This document contains automated consistency status flags. Parameters marked FAIL require engineering review before progression to Detailed Engineering.

BU-201 — SULFUR FURNACE

Datasheet Ref: DS-BU-201-001  |  Project: PRJ-SA100  |  Rev: A0 FEED

| Parameter | Specified Value / Requirement | Units / Notes | Consistency Check |
| --- | --- | --- | --- |
| Equipment type | Horizontal cylindrical combustion chamber, refractory-lined | — | Consistent with process description |
| Design code | EN 13445 (Pressure Vessels) or ASME VIII Div.1 | — | — |
| Shell material | Carbon steel SA-516 Gr.70; minimum 12 mm wall | mm | Consistent |
| Refractory lining | High-alumina castable; service temp rating ≥1,400°C; minimum 200 mm thickness | °C / mm | NOV at surge 1,080°C vs lining 1,400°C: 77% utilisation |
| Design temperature | 1,200°C (shell inside refractory surface) | °C | Adequate: surge max 1,080°C = 90% of design T |
| Design pressure | −50 mbar to +300 mbar gauge | mbar | Consistent with PSHH-201 trip at +300 mbar |
| Rated thermal input | 21,000 kW (based on sulfur LHV 9,164 kcal/kg × 1,800 kg/h max) | kW | Surge demand 18,480 kW = 88% of rating |
| Number of sulfur spray guns | 2 × 100% capacity spray guns (each rated 1,800 kg/h) | — | Each gun must handle surge at 1,499 kg/h = 83.3% of rating |
| Gun material | 316SS; gun body + nozzle; replaceable tip in Hastelloy C-276 | — | Acid-resistant; consistent with sulfur service |
| Flame detection | 2 × UV flame scanners (1oo2 voting); fail-safe on loss of signal | — | Consistent with SIF start-up permissive |
| Explosion relief panels | Required: 0.1 barg rupture disk; vent to safe location | — | Final area TBD at site layout stage |
| Inspection access | Minimum 600 mm manway; inspection port at burner end and tail end | — | Required for annual refractory inspection |
| Refractory anchor design | V-type anchors SA-240 304 SS at 150 mm pitch; explosion-proof design | — | — |
| VENDOR CONFIRMATION REQUIRED | Confirm rated thermal input and gun capacity at 1,499 kg/h surge with refractory T profile | — | MARGIN: surge T approaches 90% of design envelope |

WHB-201 — WASTE HEAT BOILER

Datasheet Ref: DS-WHB-201-001  |  Project: PRJ-SA100  |  Rev: A0 FEED

| Parameter | Specified Value / Requirement | Units / Notes | Consistency Check |
| --- | --- | --- | --- |
| Type | Fire-tube boiler (process gas in tubes, steam on shell side) | — | Consistent with process design |
| Design code | ASME I (Power Boilers) or EN 12952 | — | — |
| Tube material | SA-106 Gr.B (carbon steel, seamless); minimum 4 mm wall | mm | Consistent with gas service |
| Shell material | SA-516 Gr.70 | — | — |
| Gas inlet temperature | 1,050°C normal; 1,080°C surge; 1,200°C design maximum | °C | Surge 1,080°C = 90% of design T |
| Gas outlet temperature | 420°C target (±10°C) | °C | Consistent with converter inlet requirement |
| Rated steam capacity | 9,500 kg/h at 40 barg / 400°C (minimum) | kg/h | Surge demand 8,250 kg/h = 86.8% of rating |
| Steam drum MAWP | 48 barg (minimum) | barg | SIS trip at 46 barg; PSV at 47.5 barg; MAWP 48 barg — consistent |
| BFW inlet conditions | 105°C, 45 barg minimum (pump delivery) | °C / barg | Consistent with P-201 pump specification |
| Tube-side gas velocity | ≤5.0 m/s at rated flow (erosion limit) | m/s | MARGIN: surge velocity 4.6 m/s = 92% of limit |
| Soot blowing | Steam lance soot blowing; minimum 1 lance per 3 m tube length | — | Required for sulfur combustion products |
| Thermal expansion | Floating tube sheet or U-tube design to accommodate 630°C ΔT | — | Required; fixed tubesheet NOT acceptable |
| Three-element level control nozzles | 3 × nozzles for LT (2 × LT; 1 × level gauge); steam flow nozzle (FT-202); BFW inlet nozzle | — | Consistent with LIC-201 three-element scheme |
| VENDOR CONFIRMATION REQUIRED | Tube velocity at surge; erosion analysis; thermal cycling fatigue life (min 20 years) | — | MARGIN items must be vendor confirmed |

R-401 — DCDA CATALYTIC CONVERTER

Datasheet Ref: DS-R-401-001  |  Project: PRJ-SA100  |  Rev: A0 FEED

| Parameter | Specified Value / Requirement | Units / Notes | Consistency Check |
| --- | --- | --- | --- |
| Type | 4-pass adiabatic catalytic converter; vertical cylindrical pressure vessel | — | — |
| Shell material | SA-516 Gr.70 carbon steel; minimum 16 mm wall (due to size) | mm | — |
| Internal components | 316L SS trays, gas distributors, catalyst support grids; expansion joints between passes | — | Consistent with gas temperature range |
| Design temperature | 700°C (Pass 1 outlet — worst case) | °C | Surge actual 621°C = 88.7% of design T |
| Design pressure | 0.5 barg positive; −100 mbar vacuum design | barg | Consistent with process |
| Catalyst type (Pass 1) | Cs-promoted V₂O₅ (e.g. Haldor Topsoe VK-701 or equivalent); low ignition temp ≥380°C | — | Consistent with start-up procedure P3 (400°C minimum) |
| Catalyst type (Pass 2–4) | Standard V₂O₅/K₂S₂O₇ promoted (VK-38 or equivalent); ignition temp ≥400°C | — | — |
| Catalyst bed depths | P1: 1.2 m; P2: 0.8 m; P3: 0.8 m; P4: 1.0 m (subject to final vendor optimisation) | m | MARGIN: P1 depth may need 5% reduction per H&MB FAIL |
| Space velocity at design | P1: 550 h⁻¹; P2–P4: per catalyst vendor calculation at 100 t/d | h⁻¹ | Max 700 h⁻¹; surge 600 h⁻¹ = 85.7% of max |
| Bypass valve nozzles | P1 cold bypass: DN300 nozzle; P2/P3/P4 bypass: DN200 nozzle each | — | Must accommodate bypass flow for temperature control per TIC-401 to TIC-404 |
| Catalyst life | Minimum guaranteed 5 years at design operating conditions | yr | Vendor guarantee required; surge operation must be within guarantee scope |
| CRITICAL VENDOR ACTION | P1 catalyst bed height must be reduced by ~5% (from 1.2 m to ~1.14 m) to prevent surge temperature exceeding 620°C TSHH. Vendor to recalculate and confirm. | — | FAIL — outstanding from H&MB; purchase spec cannot be finalised until resolved |

P-702A/B — PRODUCT DISPATCH PUMP (DUTY + STANDBY)

Datasheet Ref: DS-P-702-001  |  Project: PRJ-SA100  |  Rev: A0 FEED

| Parameter | Specified Value / Requirement | Units / Notes | Consistency Check |
| --- | --- | --- | --- |
| Pump type | Centrifugal, horizontal, close-coupled or back pull-out | — | — |
| Design code | ISO 2858 / API 685 (if magnetically coupled) or equivalent | — | — |
| Fluid | 98.5 wt% H₂SO₄ | — | — |
| Fluid temperature | 35–45°C (normal); 50°C design maximum | °C | Consistent with product spec |
| Fluid density | 1,840 kg/m³ at 35°C | kg/m³ | Used for hydraulic calculations |
| Normal duty flow | 3.0 m³/h (4,167 / 1,840 × 1.45 density factor) | m³/h | Consistent with H&MB 100% load |
| RATED pump flow (REVISED) | 4.5 m³/h (upsized from previous 3.0 — per FAIL resolution) | m³/h | Previous 3.0 m³/h was FAIL at surge. Now sized at 4.5 m³/h normal (130% = 5.85 m³/h rated). Surge demand 5,417 / 1,840 = 2.95 m³/h — well within new rating. |
| Differential head | 30 m minimum (includes line losses to loading arm) | m H₂O | — |
| Motor power (REVISED) | 30 kW minimum (upsized from 22 kW with pump) | kW | 22 kW was marginal at surge; 30 kW provides adequate margin |
| Seal type | Double mechanical seal, pressurised barrier fluid (Plan 53B) | — | Required for H₂SO₄ — no single seal permitted on acid service |
| Wetted materials | Impeller: Saramet 23 (high-Si iron) or Hastelloy B-3; Casing: high-Si cast iron | — | Consistent with 98.5% H₂SO₄ service |
| Standby arrangement | Auto-start on duty pump low-flow alarm or manual selection | — | Consistent with DCS start/stop logic |
| VENDOR CONFIRMATION | Hydraulic performance test (API 610 Type A) required before shipment | — | All FAIL items resolved by resizing |

K-301A/B — MAIN PROCESS AIR BLOWER (DUTY + STANDBY)

Datasheet Ref: DS-K-301-001  |  Project: PRJ-SA100  |  Rev: A0 FEED

| Parameter | Specified Value / Requirement | Units / Notes | Consistency Check |
| --- | --- | --- | --- |
| Type | Centrifugal single-stage radial blower with Variable Frequency Drive | — | — |
| Inlet gas | Atmospheric air; moisture ≤ 25 g/Nm³ (pre-drying tower) | — | DT is downstream of blower; DT handles drying |
| Rated inlet flow | 11,000 Nm³/h at site conditions (50 m ASL, 45°C ambient) | Nm³/h | Surge demand 9,760 Nm³/h = 88.7% of rated |
| Discharge pressure | 0.65 barg minimum at rated flow | barg | Normal operating: 0.45 barg; surge: 0.50 barg; rated must cover max |
| VFD frequency range | 25–60 Hz (minimum); continuous duty at 55 Hz (110% surge) confirmed | Hz | Surge operation at 55 Hz = 91.7% of max; vendor must confirm continuous duty rating at 55 Hz |
| Surge margin | Minimum 15% surge margin at all operating points (anti-surge controller) | % | Anti-surge recycle valve mandatory in scope of supply |
| Motor rating | 630 kW at rated flow; MV 11 kV, 50 Hz | kW | Surge demand 572 kW = 90.8% of motor rating |
| Impeller material | Cast aluminium alloy (air service); statically balanced to ISO 1940 G2.5 | — | — |
| Casing material | Cast iron or fabricated steel; external epoxy coating | — | — |
| Vibration monitoring | API 670 proximity probes on drive end and non-drive end; alarm + trip | — | Consistent with rotating equipment standard |
| Acoustic enclosure | Required if noise at 1 m > 85 dB(A) | dB(A) | Plant boundary limit 70 dB(A) — vendor must provide noise data |
| Performance test | ASME PTC 10 Type 2 test at factory; surge line determined at test | — | Required before shipment |
| VENDOR CONFIRMATION | Continuous duty performance curve at 55 Hz; surge line at 110% flow; motor thermal class at VFD duty | — | MARGIN items require vendor confirmation |

T-501 — INTERPASS ABSORPTION TOWER (IPAT)

Datasheet Ref: DS-T-501-001  |  Project: PRJ-SA100  |  Rev: A0 FEED

| Parameter | Specified Value / Requirement | Units / Notes | Consistency Check |
| --- | --- | --- | --- |
| Type | Vertical packed column; acid-resistant brick-lined carbon steel shell | — | — |
| Design code | ASME VIII Div.1 for pressure rating; additional brick lining per ASTM C395 | — | — |
| Shell material | SA-516 Gr.70; minimum 12 mm wall | mm | — |
| Lining type | Monolithic acid-resistant brick (silicate-bonded); minimum 115 mm total thickness on carbon steel shell; rubber membrane between shell and brick | mm | Critical: rubber membrane prevents acid reaching shell |
| Design gas inlet temp | 480°C (maximum; normal 420°C; surge 432°C) | °C | Surge 432°C = 90% of design T |
| Design pressure | 2.5 barg / FV (full vacuum) | barg | Normal operating 0.15 barg; adequate margin |
| Gas design flow | 11,000 Nm³/h (at rated; surge 9,760 Nm³/h) | Nm³/h | Surge 88.7% of design — confirmed consistent |
| Packing type | Ceramic Raschig rings (50 mm) or equivalent structured packing (Sulzer MellapakPlus 252.Y acid-resistant) | — | Vendor to confirm packing HETP and flooding characteristics at design conditions |
| Acid inlet nozzle | DN200 (acid distribution header); DN300 (sump discharge) | mm | Sized for 132 m³/h circulating acid at surge |
| Mist eliminator | Chevron-type acid-resistant mist eliminator at tower top; ΔP < 5 mbar at design flow | — | Required to prevent acid carryover to Pass 4 |
| Level measurement nozzles | 2 × DN50 nozzles for LT-501 (process) and LT-501B (SIL instrument on separate nozzle) | — | Consistent with two independent level instruments |
| Sampling point | DN25 sample valve at acid sump; DN25 on circulating acid line (for AT-501) | — | Consistent with AT-501 density analyser installation |
| VENDOR CONFIRMATION | Gas flooding velocity at surge (9,760 Nm³/h); HETP of packing; brick lining inspection protocol | — | MARGIN: surge gas velocity at 82.5% of flooding — confirm |

E-501 — IPAT ACID COOLER

Datasheet Ref: DS-E-501-001  |  Project: PRJ-SA100  |  Rev: A0 FEED

| Parameter | Specified Value / Requirement | Units / Notes | Consistency Check |
| --- | --- | --- | --- |
| Type | Shell and tube heat exchanger; acid on tube side; CW on shell side | — | Consistent with process — acid in tubes for corrosion control |
| Design code | ASME VIII Div.1 / TEMA R (heavy industrial service) | — | — |
| Tube material | Saramet 23 (high-Si iron) or Hastelloy C-276 (vendor option) | — | Required for concentrated H₂SO₄ at 65–95°C |
| Shell material | Carbon steel SA-516 Gr.70 (CW side) | — | — |
| Tube design pressure | 6.0 barg (acid side) | barg | Acid pump discharge: 4.0 barg max; 6.0 barg provides margin |
| Shell design pressure | 6.0 barg (CW side) | barg | CW system: 4.5 barg max; margin adequate |
| Design acid inlet temp | 100°C; acid outlet target 65°C (normal); 70°C at surge | °C | Surge outlet 70°C within design — consistent |
| CW inlet temperature | 30°C (normal); 35°C maximum (at high ambient 45°C) | °C | MARGIN: at 35°C CW inlet, cooler duty may not achieve 65°C outlet; vendor to confirm at 35°C CW |
| Rated duty | 3,200 kW (minimum at 35°C CW inlet) | kW | Surge demand 2,915 kW = 91.1% of rating at 35°C CW — consistent |
| Heat transfer area | 180 m² minimum (calculated at design fouling factor Rf = 0.0002 m²K/W each side) | m² | Vendor must confirm area includes fouling allowance in calculation |
| Fouling factors | Tube side (acid): 0.0002; Shell side (CW): 0.0002 m²K/W | m²K/W | Standard TEMA fouling factors for this service |
| Thermal expansion | Floating head (TEMA S) or U-tube design; no fixed tubesheet permitted | — | Required for acid-side temperature from 65°C to 100°C |
| VENDOR CONFIRMATION | Performance at 35°C CW inlet (high ambient case); Saramet vs Hastelloy selection and cost trade-off | — | MARGIN: CW temperature sensitivity is key selection criterion |

AT-201 — FURNACE EXIT SO₂ ANALYSER

Datasheet Ref: DS-AT-201-001  |  Project: PRJ-SA100  |  Rev: A0 FEED

| Parameter | Specified Value / Requirement | Units / Notes | Consistency Check |
| --- | --- | --- | --- |
| Type | Paramagnetic O₂ analyser (for O₂ reading) + UV photometric SO₂ analyser; combined extraction type | — | Consistent with instrument specification |
| Measurement range (SO₂) | 0–15 vol% (revised from original 0–15 — no change; but AT-401 changed 0–15 as per II FAIL correction) | vol% | NOV 10.5 vol%; alarm at 12.5 vol% = 83.3% of range — consistent |
| Measurement range (O₂) | 0–15 vol% (revised from 0–25 vol%; improved resolution) | vol% | NOV 10.5 vol%; alarm AAL 8.0 vol% = 53.3% of range — good resolution |
| Sample conditioning | Stainless steel sample probe; 316SS sample line; heated (180°C) to prevent condensation; in-line filter; cooler to bring sample to 5°C before analyser | °C | Sample conditioning critical in SO₂/SO₃ duty — all wetted parts acid-resistant |
| Response time | < 30 seconds to 90% of step change | s | Fast enough for DCS cascade ratio control on FIC-201 |
| Calibration | Minimum 2-point calibration (zero + span) via solenoid-switched calibration gas manifold; manual and automatic calibration modes | — | — |
| Alarm outputs | 4–20 mA HART; discrete alarm outputs for FAIL and maintenance mode | — | Consistent with DCS I/O specification |
| Certification | ATEX Zone 2 certified analyser cabinet; IECEx equivalent accepted | — | Required for area classification at furnace exit |
| VENDOR CONFIRMATION | Calibration gas concentration available locally (SO₂ certified reference gas); sample line trace heating spec | — | — |

# VENDOR DATASHEET CONSISTENCY SUMMARY — ALL ITEMS

| Tag / Datasheet | Key Consistency Finding | Status | Purchase Specification Action |
| --- | --- | --- | --- |
| BU-201 (Furnace) | Surge T (1,080°C) = 90% of design T (1,200°C); refractory lining must be confirmed for thermal cycling at this level | MARGIN | Include thermal cycling fatigue requirement in purchase spec |
| WHB-201 | Tube-side gas velocity 4.6 m/s vs limit 5.0 m/s at surge (92%); steam output 86.8% of rated | MARGIN | Vendor must confirm tube velocity and erosion life at 4.6 m/s continuous |
| R-401 (Converter) | P1 catalyst bed height specification must be REDUCED by ~5% before purchase spec can be finalised | FAIL | Do NOT issue purchase specification until H&MB FAIL (Pass 1 temperature) is resolved. Withhold tender issue for R-401. |
| P-702 (Dispatch pump) | Resized from 3.0 to 4.5 m³/h normal; motor from 22 to 30 kW — FAIL now resolved | PASS (after resize) | Issue purchase specification at new size; confirm availability with vendors |
| K-301 (Blower) | VFD at 55 Hz continuous (91.7% of 60 Hz); motor at 90.8% of rated at surge | MARGIN | Vendor must confirm continuous duty rating at 55 Hz; include anti-surge recycle in scope |
| T-501 (IPAT) | Gas inlet design T (480°C) provides adequate margin over surge (432°C) | PASS | No critical action; confirm packing flooding at surge flow |
| E-501 (IPAT cooler) | Cooler duty at 91.1% rated at surge; sensitive to CW inlet temperature at high ambient | MARGIN | Include 35°C CW inlet performance guarantee in purchase specification |
| AT-201 (Analyser) | Range and calibration consistent after II corrections applied | PASS | No action |
| All datasheets | Surge (110%) flows and temperatures included as mandatory vendor confirmation items | PASS | Vendor quotations must include surge performance confirmation |