FRONT-END ENGINEERING DESIGN (FEED)

100 t/d Sulfuric Acid Plant — DCDA / Elemental Sulfur

BASIS OF DESIGN (BoD)

| Attribute | Value |
| --- | --- |
| Document No. | PRJ-SA100-FEED-BOD-001 |
| Revision | A0 — Issued for Review |
| Project | 100 t/d H₂SO₄ Plant, DCDA Route |
| Date | August 2026 |
| Status | FEED — Consistency-Checked Edition |

This document contains automated consistency status flags. Parameters marked FAIL require engineering review before progression to Detailed Engineering.

# 1. PLANT CAPACITY AND THROUGHPUT

## 1.1 Production Rate — Consistency Check

The plant nameplate is 100 t/d product H₂SO₄. Surge capacity (for storage buffer and downstream demand) is 110 t/d (+10%). Equipment across the plant is rated at a minimum of 110% of nameplate to accommodate the surge, and pumps are rated to 130% of normal duty flow. Status reflects maximum operating vs. equipment rated.

| Parameter | Nom. Design (100 t/d) | Max Operating (110 t/d +10%) | Equipment Rated Capacity |
| --- | --- | --- | --- |
| Product H₂SO₄ output | 4,167 kg/h | 4,583 kg/h | 5,000 kg/h (storage pump rated) |
| Sulfur feed rate | 1,363 kg/h | 1,499 kg/h | 1,800 kg/h (Coriolis MFM rated) |
| Process air volumetric | 8,873 Nm³/h | 9,760 Nm³/h | 11,000 Nm³/h (blower rated) |
| Air blower motor power | 520 kW | 572 kW | 630 kW (motor nameplate) |
| WHB steam generation | 7,500 kg/h | 8,250 kg/h | 9,500 kg/h (WHB design) |
| Molten sulfur pump flow | 1,363 kg/h | 1,775 kg/h | 1,950 kg/h (pump rated @ 130%) |
| Product dispatch pump | 4,167 kg/h | 5,417 kg/h | 5,400 kg/h (pump rated @ 130%) |
| Annual production | 300,000 t/yr | 330,000 t/yr | 350,000 t/yr (OSBL rated) |

# 2. FEEDSTOCK SPECIFICATION — DESIGN BASIS

## 2.1 Molten Sulfur Feed Quality

Instrument and analyser ranges must encompass the full quality variation expected in the feedstock. The following checks confirm analyser ranges and trip setpoints are consistent with the specification envelope.

| Parameter | Spec Limit (guaranteed) | Typical Operating Range | Analyser / Meter Range |
| --- | --- | --- | --- |
| Purity (S content, wt%) | ≥ 99.90 | 99.90–99.97 | 99.50–100.0% (density) |
| Ash content (wt%) | ≤ 0.030 | 0.005–0.025 | 0–0.05% (filter ΔP proxy) |
| Moisture (wt%) | ≤ 0.100 | 0.010–0.080 | 0–0.15% (Karl Fischer) |
| Acidity as H₂SO₄ (wt%) | ≤ 0.003 | 0.001–0.003 | 0–0.005% (titration) |
| Selenium (ppm) | ≤ 1.0 | 0.1–0.8 | 0–2 ppm ICP (lab) |
| Arsenic (ppm) | ≤ 0.5 | 0.05–0.4 | 0–1 ppm ICP (lab) |
| Feed temperature (°C) | 135–145 | 138–143 | RTD 0–200°C |
| Feed pressure (barg) | 0.30–0.50 | 0.35–0.45 | PT 0–10 barg |

# 3. PRODUCT SPECIFICATION — DESIGN BASIS

| Parameter | Product Spec | DCS Control Target | Analyser Range | Alarm HH Setpoint |
| --- | --- | --- | --- | --- |
| H₂SO₄ concentration (wt%) | 98.0–98.5 | 98.2 ± 0.15 | 96–99.5 wt% | >98.8 wt% (MARGIN alarm) |
| Iron (Fe, ppm) | ≤ 50 | <20 typical | 0–100 ppm (ICP lab) | None in DCS |
| Chloride (Cl, ppm) | ≤ 1.0 | <0.3 typical | 0–2 ppm (IC lab) | None in DCS |
| Product temperature (°C) | 25–40 | 35 ± 3 | RTD 0–100°C | >45°C alarm HIGH |
| Oleum (free SO₃, wt%) | 20 ± 1 | 20.0 ± 0.5 | 18–22% free SO₃ | — |
| Colour Hazen | ≤ 10 | <5 typical | Lab measurement only | No online analyser |

# 4. UTILITY SPECIFICATIONS — RATED vs DEMAND

| Utility | Normal Demand (100 t/d) | Peak Demand (110 t/d surge) | System Rated Supply |
| --- | --- | --- | --- |
| Cooling water (m³/h) | 240 | 264 | 300 (CW system rated) |
| Instrument air (Nm³/h) | 120 | 132 | 180 (compressor rated) |
| Plant air (Nm³/h) | 80 | 88 | 120 (compressor rated) |
| HP steam export (kg/h) | 7,500 | 8,250 | 9,500 (WHB rated) |
| MP steam (tracing, kg/h) | 350 | 385 | 500 (MP header rated) |
| Nitrogen (Nm³/h) | 15 | 18 | 25 (N₂ supply rated) |
| Electrical power (kW, MV) | 1,850 | 2,035 | 2,500 (MV switchgear rated) |
| Cooling water ΔT (°C) | 12 | 14 | 15 (CW design ΔT limit) |

# 5. ENVIRONMENTAL LIMITS — DESIGN MARGIN

| Emission Parameter | Regulatory Limit | Design Target | Max Operating Emission (est.) |
| --- | --- | --- | --- |
| SO₂ stack (mg/Nm³, dry) | ≤ 500 | ≤ 300 | ≤ 480 (at 50% turndown) |
| SO₃ / acid mist (mg/Nm³) | ≤ 35 | ≤ 20 | ≤ 30 |
| Conversion efficiency (%) | ≥ 99.5 | ≥ 99.8 | 99.65 (at 50% turndown) |
| Stack height (m) | ≥ 60 | 65 | 65 |
| Noise at boundary (dB(A)) | ≤ 70 | ≤ 65 | 63 est. |

# 6. OPERATING MODES — RANGE CONSISTENCY

| Operating Mode | Sulfur Feed (kg/h) | Air Flow (Nm³/h) | SO₂ Concentration (vol%) | Converter T° Pass 1 (°C) |
| --- | --- | --- | --- | --- |
| Normal 100% | 1,363 | 8,873 | 10.5 | 420–600 |
| Surge 110% | 1,499 | 9,760 | 10.5 | 420–615 |
| Turndown 80% | 1,090 | 7,098 | 10.5 | 410–580 |
| Turndown 60% | 818 | 5,324 | 10.5 | 405–555 |
| Minimum stable 50% | 682 | 4,436 | 10.5 | 400–545 |
| Cold start ramp-up | 0→682 | 4,000 | 0→10.5 | Ambient→420 |
| Emergency surge* | 1,640 | 10,600 | 10.5 | 420–650 |