FRONT-END ENGINEERING DESIGN (FEED)

100 t/d Sulfuric Acid Plant — DCDA / Elemental Sulfur

HEAT AND MATERIAL BALANCE (H&MB)

Consistency-Checked: Operating Range vs Equipment Rated Capacity

| Attribute | Value |
| --- | --- |
| Document No. | PRJ-SA100-FEED-HMB-002 |
| Revision | A0 — Issued for Review |
| Project | 100 t/d H₂SO₄ Plant, DCDA Route |
| Date | August 2026 |
| Status | FEED — Consistency-Checked Edition |

This document contains automated consistency status flags. Parameters marked FAIL require engineering review before progression to Detailed Engineering.

# 1. OVERALL PLANT MASS BALANCE

## 1.1 Three-Point Mass Balance Comparison

| Stream / Component | Normal (A) 100 t/d kg/h | Surge (B) 110 t/d kg/h | Turndown (C) 50 t/d kg/h | Equip. Rated Capacity kg/h or Nm³/h |
| --- | --- | --- | --- | --- |
| Molten Sulfur Feed (S) | 1,363 | 1,499 | 682 | 1,800 (MFM rated) |
| Process Air (dry, Nm³/h) | 8,873 | 9,760 | 4,437 | 11,000 (blower) |
| Product H₂SO₄ (98.5 wt%) | 4,167 | 4,583 | 2,083 | 5,000 (pump rated) |
| HP Steam Export (kg/h) | 7,500 | 8,250 | 3,750 | 9,500 (WHB rated) |
| Stack Gas — total (Nm³/h) | 7,800 | 8,580 | 3,900 | 9,500 (stack rated) |
| Cooling Water Total (m³/h) | 240 | 264 | 120 | 300 (CW system) |
| Dilution Water to Absorption (L/h) | 220 | 242 | 110 | 500 (FCV rated) |
| BFW to WHB (kg/h) | 7,700 | 8,470 | 3,850 | 10,000 (BFW pump) |
| Oleum draw-off (optional, kg/h) | 0–500 | 0–550 | 0 | 700 (oleum pump) |

# 2. UNIT 200 — SULFUR FURNACE & WASTE HEAT BOILER

## 2.1 Furnace Mass Balance

## 2.2 Waste Heat Boiler (WHB) Heat Balance

| Parameter | Normal (A) | Surge (B) | WHB Rated |
| --- | --- | --- | --- |
| Gas inlet temperature | 1,050°C | 1,080°C | 1,200°C max |
| Gas outlet temperature | 420°C | 425°C | 380–450°C target |
| Steam generation rate | 7,500 kg/h | 8,250 kg/h | 9,500 kg/h rated |
| Steam pressure | 40 barg | 40 barg | 45 barg (design P) |
| Steam temperature | 400°C | 402°C | 420°C max |
| WHB heat duty (kW) | 16,800 | 18,480 | 21,000 kW (design) |
| BFW inlet temperature | 105°C | 105°C | 60–120°C range |
| Steam drum water level range | 40–60% | 40–60% | 20% (LSLL trip) |
| WHB tube-side velocity | 4.2 m/s | 4.6 m/s | 5.0 m/s max (erosion limit) |

# 3. UNIT 400 — DCDA CONVERTER HEAT BALANCE

## 3.1 Pass-by-Pass Mass and Heat Balance

| Pass / Section | SO₂ In (kg/h) A¦B | SO₃ Formed (kg/h) A¦B | SO₂ Out (kg/h) A¦B | T Inlet (°C) | T Outlet (°C) A¦B | Catalyst T Max Rated (°C) |
| --- | --- | --- | --- | --- | --- | --- |
| Pass 1 | 3,185 ¦ 3,504 | 2,008 ¦ 2,209 | 1,177 ¦ 1,295 | 420 | 600 ¦ 621 | 620°C TSHH limit |
| Pass 2 | 1,177 ¦ 1,295 | 726 ¦ 799 | 451 ¦ 496 | 440 | 520 ¦ 526 | 550°C TSHH limit |
| Pass 3 | 451 ¦ 496 | 318 ¦ 350 | 133 ¦ 146 | 430 | 480 ¦ 484 | 500°C TSHH limit |
| IPAT (absorption) | — | 3,052¦3,358 (SO₃ abs.) | — | — | — | — |
| Pass 4 | 133 ¦ 146 | 133 ¦ 146 | <7 ¦ <8 | 415 | 430 ¦ 432 | 450°C TSHH limit |
| FAT (absorption) | — | 133¦146 (SO₃ abs.) | — | — | — | — |
| Stack tail gas | <7 ¦ <8 | — | <7 ¦ <8 | 80 | 80 | ≤7 kg/h SO₂ target |
| Overall conversion | 99.78% ¦ 99.77% | — | — | — | — | ≥99.5% guaranteed |

# 4. UNIT 300 — AIR DRYING TOWER HEAT BALANCE

| Parameter | Normal (A) | Surge (B) | Equipment Rated |
| --- | --- | --- | --- |
| Air flow to DT (Nm³/h) | 7000 | 7,760 | 8,000 (tower internals) |
| DT gas velocity (m/s) | 1.8 | 2.0 | 2.5 (flooding limit) |
| Moisture removal (kg/h) | 58 | 64 | 100 (DT design) |
| DT circulating acid flow (m³/h) | 45 | 50 | 60 (pump rated) |
| DT acid cooler duty (kW) | 320 | 352 | 420 (cooler rated) |
| DT acid concentration (wt%) | 93–96 | 93–96 | 91–98 (tower design) |
| Mist eliminator ΔP (mbar) | 8 | 9 | 15 (design max) |

# 5. UNIT 500 — ABSORPTION SYSTEM HEAT BALANCE

| Parameter | Normal (A) | Surge (B) | Equipment Rated |
| --- | --- | --- | --- |
| IPAT SO₃ absorbed (kg/h) | 3,052 | 3,358 | 4,000 (tower design) |
| IPAT acid cooler duty (kW) | 2,650 | 2,915 | 3,200 (cooler rated) |
| IPAT CW flow (m³/h) | 76 | 84 | 95 (CW branch rated) |
| IPAT circulating acid flow (m³/h) | 120 | 132 | 150 (pump rated @ 130%) |
| FAT SO₃ absorbed (kg/h) | 133 | 146 | 250 (tower design) |
| FAT acid cooler duty (kW) | 890 | 979 | 1,100 (cooler rated) |
| FAT CW flow (m³/h) | 26 | 29 | 35 (CW branch rated) |
| Dilution water total (L/h) | 220 | 242 | 500 (FCV-501 rated) |
| IPAT acid sump retention time | >8 min | >7 min | >5 min (design minimum) |

# 6. HEAT INTEGRATION SUMMARY — KEY EXCHANGER DUTIES

| Exchanger | Service | Duty Normal (kW) | Duty Surge (kW) | Rated Duty (kW) | MTD Normal (°C) |
| --- | --- | --- | --- | --- | --- |
| WHB | Furnace gas → HP steam | 16,800 | 18,480 | 21,000 | 420 |
| Interpass HX 1-2 | Pass 1 gas → Pass 2 preheat | 1,650 | 1,815 | 2,000 | 85 |
| Interpass HX 2-3 | Pass 2 gas → Pass 3 preheat | 980 | 1,078 | 1,200 | 60 |
| Interpass HX 3-IPAT | Pass 3 gas cooling to IPAT | 1,100 | 1,210 | 1,400 | 75 |
| Interpass HX 4-FAT | Pass 4 gas cooling to FAT | 620 | 682 | 800 | 45 |
| IPAT acid cooler | Acid → CW | 2,650 | 2,915 | 3,200 | 35 |
| FAT acid cooler | Acid → CW | 890 | 979 | 1,100 | 30 |
| DT acid cooler | DT acid → CW | 320 | 352 | 420 | 25 |
| Product acid cooler | Product → CW | 420 | 462 | 520 | 20 |