FRONT-END ENGINEERING DESIGN (FEED)

100 t/d Sulfuric Acid Plant — DCDA / Elemental Sulfur

ALARM LIST

ISA-18.2 Rationalised — Setpoint vs Operating Range Consistency Check

| Attribute | Value |
| --- | --- |
| Document No. | PRJ-SA100-FEED-AL-005 |
| Revision | A0 — Issued for Review |
| Project | 100 t/d H₂SO₄ Plant, DCDA Route |
| Date | August 2026 |
| Status | FEED — Consistency-Checked Edition |

# UNIT 100 — SULFUR HANDLING ALARMS

| Alarm Tag | Description | Priority | Setpoint | NOV | Gap (NOV to SP) | ORT Req. | Time to Consequence (est.) | Deadband |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TA-101-H | Transfer Line Temp High | MED | 148°C | 140 | 8°C | 30 min | ~60 min (solidification risk) | 3°C |
| TA-101-L | Transfer Line Temp Low | MED | 130°C | 140 | 10°C | 30 min | ~30 min (solidification start) | 3°C |
| LA-101-LL | Tank Level Low-Low | HIGH | 15% | 50% | 35% | 10 min | ~2 h (based on 100 t storage) | 2% |
| LA-101-HH | Tank Level High-High | HIGH | 92% | 50% | 42% | 10 min | ~1 h (remaining volume) | 2% |

# UNIT 200 — FURNACE AND WHB ALARMS

| Alarm Tag | Description | Priority | Setpoint | NOV | Gap (NOV to SP) | ORT Req. | Time to Consequence | Deadband |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TA-201-HH | Furnace Temp High-High (ESD) | CRITICAL | 1,150°C | 1,050 | 100°C | ESD auto | <30 s (refractory damage) | 10°C |
| TA-201-H | Furnace Temp High | HIGH | 1,100°C | 1,050 | 50°C | 10 min | ~5 min (before HH trip) | 10°C |
| TA-201-L | Furnace Temp Low | MED | 950°C | 1,050 | 100°C | 30 min | ~10 min (incomplete combustion) | 15°C |
| PA-201-HH | Furnace Pressure High-High | CRITICAL | 300 mbar | −15 | 315 mbar | ESD auto | <10 s (seal failure risk) | 10 mbar |
| PA-201-H | Furnace Pressure High | HIGH | 250 mbar | −15 | 265 mbar | 10 min | ~3 min (before HH trip) | 10 mbar |
| AA-201-H | SO₂ Conc. High | HIGH | 12.0 vol% | 10.5 | 1.5 vol% | 10 min | ~5 min (catalyst overload) | 0.5% |
| AA-201-L | SO₂ Conc. Low | MED | 8.5 vol% | 10.5 | 2.0 vol% | 30 min | ~15 min (under-combustion) | 0.5% |
| AA-202-L | O₂ Conc. Low | HIGH | 8.0 vol% | 10.5 | 2.5 vol% | 10 min | ~3 min (sub-stoichiometric) | 0.5% |
| LA-201-LL | WHB Drum Level Low-Low (SIL) | CRITICAL | 20% | 50 | 30% | ESD auto | <1 min (WHB dry-fire) | 3% |
| LA-201-L | WHB Drum Level Low | HIGH | 30% | 50 | 20% | 10 min | ~5 min before LL | 3% |
| LA-201-HH | WHB Drum Level High-High | HIGH | 90% | 50 | 40% | 10 min | ~30 min at max BFW | 3% |
| PA-202-HH | WHB Steam Pressure High-High | CRITICAL | 46 barg | 40 | 6 barg | ESD auto | <60 s (PSV lift) | 1 barg |
| PA-202-H | WHB Steam Pressure High | HIGH | 43 barg | 40 | 3 barg | 10 min | ~5 min (before HH) | 1 barg |

# UNIT 300 — DRYING SYSTEM ALARMS

| Alarm Tag | Description | Priority | Setpoint | NOV | Gap | ORT | Time to Consequence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AA-301-H | Moisture High (process air) | HIGH | 0.15 g/Nm³ | 0.05 | 0.10 | 10 min | ~20 min (catalyst poisoning begins) |
| AA-302-L | DT Acid Concentration Low | MED | 91.0 wt% | 94.5 | 3.5 wt% | 30 min | ~45 min (DT performance) |
| LA-301-L | DT Acid Sump Level Low | HIGH | 25% | 50 | 25% | 10 min | ~15 min (pump cavitation) |
| TA-301-H | DT Acid Temperature High | MED | 80°C | 65 | 15°C | 30 min | ~60 min (vapour increase) |

# UNIT 400 — CONVERTER ALARMS

| Alarm Tag | Description | Priority | Setpoint | NOV | Gap | ORT | Time to Consequence | Deadband |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TA-401-HH | Conv. P1 Temp HH (SIL ESD) | CRITICAL | 620°C | 600 | 20°C | ESD auto | <30 s (catalyst sintering) | 10°C |
| TA-401-H | Conv. P1 Temp High | HIGH | 600°C | 600 | 0°C | 10 min | Immediate — NOV=setpoint! | 10°C |
| TA-401-L | Conv. P1 Temp Low | HIGH | 410°C | 420 | 10°C | 10 min | ~10 min (conversion loss) | 10°C |
| TA-402-HH | Conv. P2 Temp HH (ESD) | CRITICAL | 560°C | 520 | 40°C | ESD auto | <2 min | 10°C |
| TA-402-H | Conv. P2 Temp High | HIGH | 540°C | 520 | 20°C | 10 min | ~5 min | 10°C |
| TA-407-L | Conv. P4 Temp Low | MED | 405°C | 415 | 10°C | 30 min | ~30 min (conversion loss) | 10°C |
| AA-401-H | SO₂ to Converter High | HIGH | 12.5 vol% | 10.5 | 2.0 vol% | 10 min | ~5 min | 0.3% |
| AA-403-HH | Stack SO₂ High-High (Env/ESD) | CRITICAL | 600 mg/Nm³ | 250 | 350 | ESD+5 min delay | Regulatory breach if sustained | 50 mg/Nm³ |
| AA-403-H | Stack SO₂ High (Env) | HIGH | 500 mg/Nm³ | 250 | 250 | 10 min | ~15 min (regulatory) | 50 mg/Nm³ |

# UNIT 500 — ABSORPTION ALARMS

| Alarm Tag | Description | Priority | Setpoint | NOV | Gap | ORT | Time to Consequence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LA-501-L | IPAT Level Low | HIGH | 20% | 50 | 30% | 10 min | ~10 min (pump loss) |
| LA-501-HH | IPAT Level High-High | HIGH | 90% | 50 | 40% | 10 min | ~20 min (overflow) |
| AA-501-L | IPAT Acid Concentration Low | MED | 97.5 wt% | 98.2 | 0.7 wt% | 30 min | ~30 min (SO₃ slip) |
| AA-501-H | IPAT Acid Concentration High | MED | 98.8 wt% | 98.2 | 0.6 wt% | 30 min | ~20 min (mist risk) |
| LA-502-L | FAT Level Low | HIGH | 20% | 50 | 30% | 10 min | ~10 min |
| FA-501-L | Dilution Water Flow Low | HIGH | 50 L/h | 220 | 170 L/h | 10 min | ~8 min (acid rise) |
| TA-502-H | IPAT Acid Temp High | MED | 80°C | 65 | 15°C | 30 min | ~45 min |

# UNIT 700 AND GENERAL — ALARMS

| Alarm Tag | Description | Priority | Setpoint | NOV | Gap | ORT | Time to Consequence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LA-701-HH | Product Tank Level HH (ESD) | CRITICAL | 90% | 50% | 40% | ESD auto | <30 min (overflow at fill rate) |
| LA-701-H | Product Tank Level High | HIGH | 80% | 50% | 30% | 10 min | ~15 min before HH |
| LA-701-L | Product Tank Level Low | MED | 15% | 50% | 35% | 30 min | ~2 h at normal draw-off |
| TA-701-H | Product Acid Temp High | MED | 45°C | 35 | 10°C | 30 min | ~60 min (vapour risk) |
| AA-901-HH | Area SO₂ Evacuation HH | CRITICAL | 25 ppm | 0 | 25 ppm | Auto PA | IDLH immediate |
| AA-901-H | Area SO₂ Warning | HIGH | 5 ppm | 0 | 5 ppm | 10 min | ~5–10 min to discomfort level |