FRONT-END ENGINEERING DESIGN (FEED)

100 t/d Sulfuric Acid Plant — DCDA / Elemental Sulfur

CAUSE AND EFFECT (C&E) MATRIX

Safety Instrumented Function Logic — Setpoint & SIL Consistency Verified

| Attribute | Value |
| --- | --- |
| Document No. | PRJ-SA100-FEED-CE-006 |
| Revision | A0 — Issued for Review |
| Project | 100 t/d H₂SO₄ Plant, DCDA Route |
| Date | August 2026 |
| Status | FEED — Consistency-Checked Edition |

# 1. PROTECTION LAYER HIERARCHY VERIFICATION

For each SIF, the following hierarchy must hold: Normal Operating Value (NOV) < DCS Alarm < SIS Trip. A violation (where the trip setpoint is at or below the alarm setpoint, or the alarm setpoint is at or below NOV) represents a protection layer conflict.

| SIF ID | Service | NOV | DCS Alarm (HIGH) | SIS Trip Setpoint |
| --- | --- | --- | --- | --- |
| SIF-01 | Furnace Temperature High-High | 1,050°C | TAH 1,100°C | TSHH 1,150°C |
| SIF-02 | Furnace Pressure High-High | −15 mbar | PAH 250 mbar | PSHH 300 mbar |
| SIF-03 | WHB Steam Drum Level Low-Low | 50% | LAL 30% | LSLL 20% |
| SIF-04 | WHB Steam Drum Pressure High-High | 40 barg | PAH 43 barg | PAHH 46 barg |
| SIF-05 | Converter Pass 1 Temp High-High | 600°C | TAH 600°C | TSHH 620°C |
| SIF-06 | Stack SO₂ High-High (Environmental) | 250 mg/Nm³ | AAH 500 | AAHH 600 |
| SIF-07 | Product Tank Level High-High | 50% | LAH 80% | LSHH 90% |
| SIF-08 | Area SO₂ Monitor (F&G) | 0 ppm | AAH 5 ppm | AAHH 25 ppm |

# 2. ESD TRIP MATRIX — CAUSE AND EFFECT

Legend: ● = Trip output ACTUATED  ○ = No action  FC = Fail-Closed  FO = Fail-Open

| Initiator Tag | Initiating Condition | SIF ID | SIL | Voting | SDV-E01 Sulfur Block (FC) | SDV-E02 Air Block (FO) | SDV-E03 WHB BFW (FO) | XV-E04 Stack Damper | SDV-E05 Product Block (FC) | SV-E06 WHB Steam Trip | F&G PA Evacuation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TSHH-201 | Furnace T > 1,150°C | SIF-01 | SIL 2 | 1oo2 (dual TC) | ● | ○ | ● | ● | ○ | ○ | ○ |
| PSHH-201 | Furnace P > 300 mbar | SIF-02 | SIL 2 | 1oo1 | ● | ● | ○ | ● | ○ | ○ | ○ |
| LSLL-201 | WHB Level < 20% | SIF-03 | SIL 1 | 1oo1 | ● | ○ | ○ | ○ | ○ | ● | ○ |
| PAHH-202 | WHB Steam P > 46 barg | SIF-04 | SIL 2 | 1oo1 | ● | ○ | ● | ● | ○ | ● | ○ |
| TSHH-401 | Conv. P1 T > 620°C | SIF-05 | SIL 2 | 1oo2 | ● | ● | ○ | ○ | ○ | ○ | ○ |
| AT-403-HH | Stack SO₂ > 600 mg/Nm³ | SIF-06 | SIL 1 (Env) | 1oo1 + 5 min delay | ● | ○ | ○ | ● | ○ | ○ | ○ |
| LSHH-701 | Product Tank > 90% | SIF-07 | SIL 2 | 1oo2 | ○ | ○ | ○ | ○ | ● | ○ | ○ |
| Manual ESD | Panel/Field ESD button | — | — | Manual (latching) | ● | ● | ○ | ● | ● | ○ | ● |
| AA-901-HH | Area SO₂ > 25 ppm (F&G) | SIF-08 | F&G | 1oo1 | ● | ● | ○ | ○ | ○ | ○ | ● |
| IA Failure | IA Pressure < 5 barg | Utility | — | 1oo1 | ● (FC spring) | — (FO spring) | — (FO spring) | — | ● (FC spring) | — | — |

# 3. FAIL-SAFE VALVE POSITION CONSISTENCY CHECK

Every ESD valve fail-safe position must be consistent with the process safe state. The check below confirms the fail-safe position is correctly defined and consistent across the C&E matrix, P&ID (to be issued), and valve datasheets.

| Valve Tag | Service | Fail-Safe Position | Process Safe State Required |
| --- | --- | --- | --- |
| SDV-E01 | Sulfur Block Valve | Fail Closed (FC) | Isolate fuel supply — FC correct |
| SDV-E02 | Air Block Valve | Fail Open (FO) | Maintain air purge on ESD — FO correct |
| SDV-E03 | WHB BFW Block Valve | Fail Open (FO) | Protect WHB from dry-fire on IA loss — FO |
| XV-E04 | Stack Gas Damper (isolation) | Fail Closed (FC) | Open on ESD for emergency vent — should be FO |
| SDV-E05 | Product Block Valve | Fail Closed (FC) | Contain product on shutdown — FC correct |
| SV-E06 | WHB Steam Trip Valve | Fail Closed (FC) | Isolate steam on WHB dry-fire trip — FC correct |
| FCV-101 | Sulfur Feed Control Valve | Fail Closed (FC) | BPCS: isolate fuel on control failure — FC |
| FCV-201 | Air Blower VFD control | Fail at last then zero | BPCS: maintain air flow stability — correct |
| FCV-501 | Dilution Water Valve | Fail Closed (FC) | BPCS: prevent excess dilution on IA failure — FC |
| LCV-201 | WHB BFW Control Valve | Fail Open (FO) | BPCS: maximum cooling protection — FO correct |
| TCV-501 | IPAT CW Control Valve | Fail Open (FO) | BPCS: maximum cooling on failure — FO correct |

# 4. START-UP PERMISSIVE MATRIX — CONSISTENCY CHECK

| Permissive ID | Tag | Condition | Linked to DCS? | Linked to Instrument Index? |
| --- | --- | --- | --- | --- |
| P-01 | TT-201 | Furnace T < 600°C (cold start) | Yes | Yes — TT-201 range 500–1,200°C |
| P-02 | AT-301 | Air moisture < 0.15 g/Nm³ | Yes | Yes — AT-301 range 0–0.5 g/Nm³ |
| P-03 | LT-201 | WHB drum level 40–70% | Yes | Yes — LT-201 range 0–100% |
| P-04 | PT-201 | Furnace P < 50 mbar | Yes | Yes — PT-201 range −50 to +500 |
| P-05 | LT-501 | IPAT sump level 40–60% | Yes | Yes — LT-501 range 0–100% |
| P-06 | LT-502 | FAT sump level 40–60% | Yes | Yes — LT-502 range 0–100% |
| P-07 | AT-501 | IPAT acid 98.0–98.5 wt% | Yes | Yes — AT-501 range 96–99 wt% |
| P-08 | AT-502 | FAT acid 98.0–98.5 wt% | Yes | Yes — AT-502 range 96–99 wt% |
| P-09 | FT-201 | Air flow > 3,000 Nm³/h | Yes | Yes — FT-201 range 0–15,000 |
| P-10 | LT-101 | Sulfur tank level > 30% | Yes | Yes — LT-101 range 0–100% |
| P-11 | — | ESD system healthy | Yes — SIS readback | SIS to DCS hardwired |
| P-12 | — | DCS in AUTO mode | Yes — operator confirm | N/A |