FRONT-END ENGINEERING DESIGN (FEED)

100 t/d Sulfuric Acid Plant — DCDA / Elemental Sulfur

CONTROL PHILOSOPHY

PID Loop Configuration & Control Range Consistency Verified

| Attribute | Value |
| --- | --- |
| Document No. | PRJ-SA100-FEED-CP-007 |
| Revision | A0 — Issued for Review |
| Project | 100 t/d H₂SO₄ Plant, DCDA Route |
| Date | August 2026 |
| Status | FEED — Consistency-Checked Edition |

# 1. MASTER PRODUCTION RATE GOVERNOR

A "Plant Load" setpoint (expressed as % of 100 t/d nameplate, range: 50–110%) entered by the operator at the DCS cascades to all unit production-rate-dependent controllers. The Governor calculates the required sulfur mass flow and air volumetric flow targets for the current load and passes them as remote setpoints to FIC-101 and FIC-201 respectively, with lead/lag sequencing (air leads on load increase; sulfur leads on load reduction).

| Parameter | Normal (100%) | Surge (110%) | Turndown (50%) | Governor Range (DCS) |
| --- | --- | --- | --- | --- |
| Plant Load setpoint (%) | 100% | 110% | 50% | 50–110% (hard cap) |
| SO₂ target from Governor (vol%) | 10.5 | 10.5 | 10.5 | Fixed ratio 10.5% |
| Sulfur feed target from Gov. (kg/h) | 1,363 | 1,499 | 682 | FIC-101 range 0–2,000 |
| Air flow target from Gov. (Nm³/h) | 8,873 | 9,760 | 4,437 | FIC-201 range 0–15,000 |
| Load ramp rate (% per min) | 5%/min | 5%/min | 5%/min | Fixed DCS limit |

# 2. REGULATORY PID CONTROL LOOPS — CONFIGURATION CONSISTENCY

| Loop ID | Service | PV Tag | MV Tag | Control Mode | SP (Normal) | MV @ Normal (% output) | MV @ Surge (110%) | MV @ Turndown (50%) | Actuator Rated Range | MV Saturated at Surge? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FIC-101 | Sulfur Feed Rate | FT-101 (Coriolis) | FCV-101 (modulating valve) | Cascade from Governor; PID | 1,363 kg/h | 68% | 75% | 34% | FCV-101 Cv 0–100% | No |
| FIC-201 | Air Flow / SO₂ Ratio | AT-201 (SO₂ analyser) | Blower VFD (FCV-201) | Cascade; primary SO₂; secondary air flow | 10.5 vol% | 100% VFD at 50 Hz | 110% VFD at 55 Hz | 52% VFD at 26 Hz | VFD 0–60 Hz | VFD at 91.7% of rated max Hz |
| TIC-401 | Conv. P1 Inlet Temperature | TT-401 | TCV-401 (cold bypass) | Split-range PID | 420°C | 45% bypass | 52% bypass | 22% bypass | 0–100% valve open | No |
| TIC-402 | Conv. P2 Inlet Temperature | TT-403 | TCV-402 (bypass) | PID Auto | 440°C | 40% | 44% | 20% | 0–100% | No |
| TIC-403 | Conv. P3 Inlet Temperature | TT-405 | TCV-403 (bypass) | PID Auto | 430°C | 38% | 42% | 18% | 0–100% | No |
| TIC-404 | Conv. P4 Inlet Temperature | TT-407 | TCV-404 (bypass) | PID Auto | 415°C | 35% | 38% | 17% | 0–100% | No |
| AIC-501 | IPAT Acid Concentration | AT-501 (density) | FCV-501 (water valve) | Cascade PID (acid conc. → water flow) | 98.2 wt% | 44% FCV open | 48% FCV open | 22% FCV open | FCV 0–100% (0–500 L/h) | No |
| AIC-502 | FAT Acid Concentration | AT-502 (density) | FCV-502 (water valve) | Cascade PID | 98.2 wt% | 44% | 48% | 22% | 0–100% | No |
| LIC-201 | WHB Steam Drum Level (3-element) | LT-201 + FT-202 + BFW FT | LCV-201 (BFW valve) | 3-element control: Level + steam flow + BFW flow feed-forward | 50% level | 65% | 72% | 33% | 0–100% | No |
| PIC-202 | WHB Steam Pressure | PT-202 | PCV-202 (export valve) | PID Auto | 40 barg | 55% | 60% | 28% | 0–100% | No |
| LIC-501 | IPAT Sump Level | LT-501 | LCV-501 (acid transfer) | PID Auto | 50% | 50% | 55% | 25% | 0–100% | No |
| LIC-502 | FAT Sump Level | LT-502 | LCV-502 (acid transfer) | PID Auto | 50% | 50% | 55% | 25% | 0–100% | No |
| TIC-501 | IPAT Acid Temperature | TT-502 | TCV-501 (CW to acid cooler) | PID Auto | 65°C | 60% | 68% | 30% | 0–100% (FO fail-safe) | No |
| FIC-701 | Product Dispatch Flow | FT-701 (Coriolis) | Dispatch pump VFD | Batch PID | Per order kg/h | Variable | 85% pump VFD | N/A | Pump VFD 0–100% | No |

# 3. CASCADE AND RATIO LOOP CONSISTENCY CHECK

Cascade loops require that the primary (outer) loop responds significantly slower than the secondary (inner) loop. The ratio check confirms that the ratio of primary-to-secondary time constants is ≥5:1 for stable cascade operation.

| Cascade Pair | Primary Loop / PV | Secondary Loop / PV | Primary Time Const. (est.) | Secondary Time Const. (est.) | Ratio P:S |
| --- | --- | --- | --- | --- | --- |
| FIC-201 (Air/SO₂ ratio) | AT-201 SO₂ analyser (slow — analyser lag) | FT-201 Air flow (fast — DP transmitter) | ~2–5 min (analyser + process lag) | ~5–15 s (flow loop) | ~10:1 |
| AIC-501 (IPAT acid conc.) | AT-501 density (moderate response) | FCV-501 water flow (fast — magnetic FM) | ~3–8 min (dilution mixing) | ~10–30 s (flow) | ~8:1 |
| AIC-502 (FAT acid conc.) | AT-502 density | FCV-502 water flow | ~3–8 min | ~10–30 s | ~8:1 |
| LIC-201 (3-element WHB) | LT-201 drum level (slow — integrating process) | BFW flow + steam flow (fast feed-forward) | Integrating — no time const. | ~5–10 s | N/A — feed-forward + FB |
| TIC-401 P1 bypass (split-range) | TT-401 inlet temp | TCV-401 bypass valve (split-range) | ~1–3 min (gas residence time) | ~5–20 s (valve stroke) | ~10:1 |

# 4. CONTROL LOOP INTERACTION CHECK

In a DCDA plant, several control loops interact. The matrix below identifies significant interactions and assesses whether the decoupling or sequencing strategy is adequate.

| Loop Pair | Interaction Description | Interaction Severity | Decoupling Strategy | Adequately Addressed? |
| --- | --- | --- | --- | --- |
| FIC-101 ↔ FIC-201 | Sulfur feed and air flow co-determine SO₂ concentration. Changing either affects the other. | HIGH | Governor ratio control: air leads on increase, sulfur leads on decrease (lead/lag) | Yes — Governor correctly sequences the changes |
| AIC-501 ↔ AIC-502 | Both absorption towers share a common product acid stream; concentration changes in IPAT circuit affect FAT circuit if acid is cross-transferred. | MEDIUM | Independent circuits with separate dilution water controls; only product overflow interconnects them | Yes — circuits are independent at FEED; confirm with detailed piping layout |
| LIC-201 ↔ PIC-202 | WHB drum level and steam header pressure interact: rapid steam header pressure drop causes drum level swell (shrink-and-swell phenomenon). | HIGH | 3-element control on LIC-201 compensates for steam flow demand; PIC-202 uses steam let-down valve | Yes — but shrink/swell transient must be validated by dynamic simulation |
| TIC-401 ↔ TIC-402 | Pass 1 outlet temperature (TIC-401 bypass) directly affects Pass 2 inlet temperature (TIC-402). These loops are strongly coupled. | HIGH | Interpass heat exchangers are fixed-area; bypass valves are independently controlled. Detuned PID gains required for TIC-402. | MARGIN — cascade from TIC-401 output to TIC-402 setpoint should be considered to decouple the interaction |
| FIC-201 ↔ TIC-401 | Air flow rate affects converter inlet temperature through furnace heat release and WHB outlet conditions. | MEDIUM | Furnace temperature maintained by BPCS; interpass HX provides buffer | Yes — adequate for regulatory control |