FRONT-END ENGINEERING DESIGN (FEED)

100 t/d Sulfuric Acid Plant — DCDA / Elemental Sulfur

SAFETY REQUIREMENTS SPECIFICATION (SRS) & SIL RECORDS

IEC 61511 / IEC 61508 — FEED Stage SIL Determination & Consistency Check

| Attribute | Value |
| --- | --- |
| Document No. | PRJ-SA100-FEED-SRS-008 |
| Revision | A0 — Issued for Review |
| Project | 100 t/d H₂SO₄ Plant, DCDA Route |
| Date | August 2026 |
| Status | FEED — Consistency-Checked Edition |

# 2. SIL DETERMINATION — RISK GRAPH METHOD

## 2.1 Risk Graph Parameters

The IEC 61511 Risk Graph uses four parameters: C = Consequence severity; F = Frequency/exposure of personnel; P = Probability of avoiding harm; W = Demand rate (without SIS). These parameters combine to yield a SIL target.

| Parameter | Level | Definition Used in This Assessment |
| --- | --- | --- |
| C — Consequence | C1 | Minor injury; first-aid level |
| C — Consequence | C2 | Serious irreversible injury to one or more persons; possible fatality |
| C — Consequence | C3 | Likely fatality to one or more persons |
| C — Consequence | C4 | Multiple fatalities |
| F — Frequency | F1 | Rare to more frequent exposure (<1 h/d in hazardous area) |
| F — Frequency | F2 | Frequent to continuous exposure (>1 h/d) |
| P — Avoidance | P1 | Possible under specified conditions (alarm, evacuation route clear) |
| P — Avoidance | P2 | Barely possible (fast hazard development, confined space) |
| W — Demand rate | W1 | Very low (<0.01/yr without SIS) |
| W — Demand rate | W2 | Low (0.01–0.1/yr without SIS) |
| W — Demand rate | W3 | High (>0.1/yr without SIS) |

## 2.2 SIF Risk Graph Assessment

| SIF ID | SIF Description | Hazardous Event | C | F | P | W | SIL Target (Risk Graph) | SIL Assigned | Cross-Doc Consistency |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SIF-01 | Sulfur Furnace Temperature HH | Furnace over-temperature → refractory failure → SO₂/S vapour release → fire/explosion | C3 | F2 | P2 | W2 | SIL 2 | SIL 2 | Matches C&E Doc 6; TSHH-201 range issue flagged in II Doc 4 |
| SIF-02 | Furnace Pressure HH | Over-pressure → furnace seal/flange failure → SO₂ release → toxic cloud | C3 | F2 | P2 | W2 | SIL 2 | SIL 2 | Consistent across C&E, II, Alarm List |
| SIF-03 | WHB Steam Drum Level LL | Dry-fire → WHB tube rupture → steam explosion; high-pressure steam release | C3 | F1 | P1 | W2 | SIL 1 | SIL 1 | Consistent |
| SIF-04 | WHB Steam Drum Pressure HH | Over-pressure → PSV lift → steam release; if PSV fails → vessel rupture | C3 | F1 | P2 | W2 | SIL 2 | SIL 2 | Consistent; 3 barg alarm→trip gap flagged as MARGIN in C&E |
| SIF-05 | Converter Pass 1 Temp HH | Catalyst over-temp → catalyst sintering → conversion loss → SO₂ emission breach + process loss | C2 | F1 | P1 | W2 | SIL 1 | SIL 2 | FAIL: SIL 2 assigned but NOV=alarm setpoint. SIF integrity compromised by process design issue. Resolution mandatory before SRS can be approved. |
| SIF-06 | Stack SO₂ Emission HH | Regulatory emission limit breach → environmental impact; potential community evacuation | C2 | F2 | P1 | W2 | SIL 1 | SIL 1 (Environmental) | Consistent; 5-min delay timer must be documented in SRS |
| SIF-07 | Product Tank Level HH | Tank overflow → large H₂SO₄ spill → bund activation; personnel burn injury | C3 | F1 | P1 | W2 | SIL 1 | SIL 2 | SIL 2 assigned vs SIL 1 required — conservative; acceptable per IEC 61511 Cl. 3.2.4 |
| SIF-08 | Area SO₂ F&G Monitor HH | Toxic SO₂ concentration > IDLH → personnel fatality if undetected | C4 | F2 | P2 | W1 | SIL 2 | F&G (non-SIL) | F&G system classified separately per IEC 61511; not a SIS SIF. Ensure F&G coverage map reviewed at HAZOP. |

# 3. SAFETY REQUIREMENTS SPECIFICATION — SIF BY SIF

## SIF-01: Sulfur Furnace Temperature High-High

| SRS Attribute | Requirement |
| --- | --- |
| SIF ID / Tag | SIF-01 / TSHH-201 |
| SIF Description | Shutdown sulfur feed and initiate plant ESD on high furnace temperature |
| SIL Target | SIL 2 |
| Process Demand Condition | Furnace temperature exceeds 1,150°C |
| Safe State | SDV-E01 Closed (sulfur isolated); steam trip SV-E06 closed; stack damper open (FO) |
| Final Element(s) | SDV-E01 (FC, spring-return); SV-E06 (FC, spring-return) |
| Initiator(s) | TSHH-201A and TSHH-201B (dual Type B thermocouples, 500–1,300°C range — see II FAIL resolution) |
| Voting Logic | 1oo2 — either TC reaching setpoint initiates trip (diverse, independent loops) |
| Trip Setpoint | 1,150°C |
| Reset | Manual reset only at SIS panel; operator confirm + Process Engineer authorisation |
| Response Time | Valve SDV-E01 to closed position ≤ 2 seconds from solenoid de-energisation |
| Proof Test Interval | 12 months (recommended); PFD to be calculated at DE stage |
| Architecture Required | Minimum 1oo2 (IEC 61508 HFT=1) to achieve SIL 2 with single-channel sensors |
| Functional Test Method | Trip test via SIS bypass switch with DCS permissive; verify valve closure by position feedback |
| Bypasses / Inhibits | One bypass permitted during maintenance; second TC must remain active; time-limit 8 h; logged |
| SRS Consistency Status | MARGIN — TC range must be changed to 500–1,300°C (see Instrument Index FAIL item) |

## SIF-02: Furnace Pressure High-High

| SRS Attribute | Requirement |
| --- | --- |
| SIF ID / Tag | SIF-02 / PSHH-201 |
| SIL Target | SIL 2 |
| Process Demand Condition | Furnace chamber gauge pressure exceeds +300 mbar |
| Safe State | SDV-E01 Closed (sulfur); SDV-E02 Open (air continues for purge); stack damper open |
| Initiator(s) | PSHH-201 — dedicated SIL-rated pressure transmitter (−50 to +500 mbar) |
| Voting Logic | 1oo1 — single instrument; consider upgrading to 2oo3 at DE stage for SIL 2 proof |
| Trip Setpoint | +300 mbar gauge |
| Architecture Required | 1oo1 with high diagnostic coverage (DC ≥ 90%); SFF ≥ 90% required for Type B SIL 2 |
| SRS Consistency Status | PASS — setpoint at 64% of transmitter span; meets IEC 61511 range criterion |

## SIF-03: WHB Steam Drum Level Low-Low

| SRS Attribute | Requirement |
| --- | --- |
| SIF ID / Tag | SIF-03 / LSLL-201 |
| SIL Target | SIL 1 |
| Process Demand Condition | WHB steam drum level falls below 20% of normal level gauge range |
| Safe State | SDV-E01 Closed (sulfur isolated — removes heat source); SV-E06 steam trip closed |
| Initiator(s) | LSLL-201 — guided-wave radar level transmitter, 0–100% range, SIL 1 rated |
| Voting Logic | 1oo1 |
| Trip Setpoint | 20% |
| Response Time | Within 3 seconds of setpoint reached |
| SRS Consistency Status | PASS |

## SIF-04: WHB Steam Drum Pressure High-High

| SRS Attribute | Requirement |
| --- | --- |
| SIF ID / Tag | SIF-04 / PAHH-202 |
| SIL Target | SIL 2 |
| Process Demand Condition | Steam drum pressure exceeds 46 barg |
| Safe State | SDV-E01 Closed; LCV-201 BFW valve fail-open (increase cooling); SV-E06 steam trip closed |
| Initiator(s) | PAHH-202 — dedicated pressure transmitter 0–50 barg, SIL 2 rated |
| Voting Logic | 1oo1; recommend upgrade to 2oo3 at DE for SIL 2 confidence |
| Trip Setpoint | 46 barg (MAWP = 48 barg; PSV set at 47.5 barg as final mechanical layer) |
| Mechanical Protection | PSV set at 47.5 barg — mandatory as independent final layer; confirms LOPA credit |
| SRS Consistency Status | MARGIN — gap between DCS HIGH alarm (43 barg) and SIS trip (46 barg) is 3 barg; validated by dynamic simulation required |

## SIF-05: Converter Pass 1 Temperature High-High

| SRS Attribute | Requirement |
| --- | --- |
| SIF ID / Tag | SIF-05 / TSHH-401 |
| SIL Target | SIL 2 |
| Process Demand Condition | Converter Pass 1 outlet temperature exceeds 620°C |
| Safe State | SDV-E01 Closed (sulfur isolated); SDV-E02 Open (air purge continues) |
| CRITICAL SRS CONFLICT | At 110% surge load the Pass 1 outlet temperature reaches 621°C — 1°C above the SIS trip setpoint. This means: (a) the SIF will demand on every surge operation; (b) the protection layer hierarchy is broken (NOV = DCS alarm setpoint). This SRS record CANNOT be approved until the process design is corrected. The SIF cannot be considered functional in its current form. |
| Required Design Action | (1) Reduce Pass 1 catalyst bed height by ~5% OR increase cold bypass valve (TCV-401) maximum Cv by 20% to lower surge outlet temperature to ≤608°C. (2) Re-set: NOV = 600°C (normal) → DCS alarm TAH = 610°C → SIS trip TSHH = 625°C. (3) Update Instrument Index, Alarm List, C&E, and this SRS simultaneously under MOC. |
| Initiator(s) | TSHH-401A and TSHH-401B (dual TCs, Type K multipoint, 400–700°C range) |
| Voting Logic | 1oo2 |
| SRS Consistency Status | FAIL — SIF is non-functional at surge; SRS cannot be approved in current state |

## SIF-06: Stack SO₂ Emission High-High

| SRS Attribute | Requirement |
| --- | --- |
| SIF ID / Tag | SIF-06 / AT-403-HH |
| SIL Target | SIL 1 (Environmental Safety Function) |
| Process Demand Condition | Stack SO₂ concentration sustained above 600 mg/Nm³ for >5 minutes (confirmed high) |
| Safe State | SDV-E01 Closed (sulfur isolated); stack damper XV-E04 Open (confirmed FO after C&E FAIL correction) |
| Time Delay | 5-minute confirmed-high timer to avoid spurious trips on analyser noise; timer logic in SIS PLC |
| Initiator(s) | AT-403 — UV-DOAS CEMS analyser, 0–1,000 mg/Nm³, certified to EN 14181 (QAL1) |
| QAL Requirements | Annual QAL2 calibration required; 3-point calibration gas certified traceable to NIST/NPL |
| SRS Consistency Status | PASS |

## SIF-07: Product Tank Level High-High

| SRS Attribute | Requirement |
| --- | --- |
| SIF ID / Tag | SIF-07 / LSHH-701 |
| SIL Target | SIL 2 (conservative; risk graph requires SIL 1 minimum) |
| Process Demand Condition | Product acid tank level exceeds 90% of gauge range |
| Safe State | SDV-E05 Closed (product isolation); pump stopped; tanker loading halted |
| Initiator(s) | LT-702 — guided-wave radar (SIL 2 rated), 0–100%; independent of process LT-701 |
| Voting Logic | 1oo2 (LT-701 + LT-702 — two independent level transmitters on separate nozzles) |
| SRS Consistency Status | PASS |

# 4. SIS ARCHITECTURE REQUIREMENTS

| Requirement | Specification | Standard Reference |
| --- | --- | --- |
| SIS hardware platform | Dedicated Safety PLC (TÜV-certified SIL 2 capable); e.g. Siemens S7-F, Rockwell GuardLogix, or Hima HIMax | IEC 61511-1 Cl. 11 |
| SIS / DCS separation | Physical and electrical separation; DCS read-only access to SIS I/O via hardwired or OPC-UA (IEC 62443 secured) | IEC 61511-1 Cl. 11.2 |
| Power supply | Dual redundant UPS-backed 24 VDC SIS power; minimum 1-hour battery autonomy on utility loss | IEC 61511-1 Cl. 11.4 |
| Solenoid valve type | Energise-to-trip (de-energised = safe state) for all ESD valves; spring-return actuators | IEC 61511-1 Cl. 11.5 |
| Proof test coverage | Minimum 90% proof test coverage (PTC) for all SIF elements to meet SIL 2 PFD targets | IEC 61508-2 |
| Spurious trip rate target | STR < 1 per 2 years per SIF (plant availability requirement) | IEC 61511-1 Cl. 11.9 |
| Bypass management | Hardwired key-switch bypass with automatic timer (maximum 8 h); alarm to DCS and SIS log | ISA-84 |
| SIS logic solver redundancy | 1oo1D (simplex with diagnostics) minimum for SIL 1; 1oo2 or 2oo3 for SIL 2 functions | IEC 61508-6 |
| Common-cause failure (CCF) | Beta factor (β) = 10% minimum for duplicated sensors; diverse technology where feasible | IEC 61508-6 Annex D |
| Cybersecurity | SIS network airgapped from corporate IT; IEC 62443 Zone-and-Conduit model | IEC 62443 |
| FAT/SIT requirements | Full Factory Acceptance Test per IEC 61511-1 Cl. 18 before site installation | IEC 61511-1 Cl. 18 |

# 5. SIL RECORD SUMMARY — CONSISTENCY CHECK

| SIF | Description | SIL Target | SIL Assigned | Risk Graph Consistent? | Cross-Doc Consistent? | SRS Approvable? |
| --- | --- | --- | --- | --- | --- | --- |
| SIF-01 | Furnace T HH | SIL 2 | SIL 2 | Yes | MARGIN (TC range) | Conditional — after TC range change |
| SIF-02 | Furnace P HH | SIL 2 | SIL 2 | Yes | Yes | Yes |
| SIF-03 | WHB Level LL | SIL 1 | SIL 1 | Yes | Yes | Yes |
| SIF-04 | WHB Steam P HH | SIL 2 | SIL 2 | Yes | MARGIN | Conditional — dynamic sim required |
| SIF-05 | Conv. P1 T HH | SIL 2 | SIL 2 | Yes | FAIL | NO — process design must be corrected |
| SIF-06 | Stack SO₂ HH | SIL 1 | SIL 1 | Yes | Yes | Yes |
| SIF-07 | Product Tank L HH | SIL 1 | SIL 2 | Conservative | Yes | Yes (conservative SIL acceptable) |
| SIF-08 | Area SO₂ F&G | N/A (F&G) | F&G | N/A | MARGIN | Conditional — confirm F&G coverage |