FRONT-END ENGINEERING DESIGN (FEED)

100 t/d Sulfuric Acid Plant — DCDA / Elemental Sulfur

LINE LIST (PIPING LINE REGISTER)

ASME B31.3 / ASME B31.1 — Design Conditions vs Fluid Service Consistency Check

| Attribute | Value |
| --- | --- |
| Document No. | PRJ-SA100-FEED-LL-009 |
| Revision | A0 — Issued for Review |
| Project | 100 t/d H₂SO₄ Plant, DCDA Route |
| Date | August 2026 |
| Status | FEED — Consistency-Checked Edition |

# PIPE CLASS SUMMARY

| Pipe Class | Material | Fluid Service | Design P (barg) | Design T (°C) | Wall Thk Basis | Corr. Allow. | Lining/Coating |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | Carbon Steel ASTM A106 Gr.B | Steam, BFW, utilities | 50 | 450 | ASME B31.1 | 1.5 mm | None |
| A2 | Carbon Steel ASTM A106 Gr.B | Process air, N₂ | 2 | 60 | ASME B31.3 | 1.5 mm | External epoxy |
| B1 | Stainless Steel 316L | SO₂ / SO₃ hot gas | 0.5 | 650 | ASME B31.3 | 0 mm | None |
| B2 | Carbon Steel + brick lining | H₂SO₄ process lines | 2 | 120 | ASME B31.3 | 4 mm brick | Acid brick (4 mm) + rubber membrane |
| B3 | Saramet / Hastelloy C-276 | H₂SO₄ cooler circuits | 10 | 100 | ASME B31.3 | 0 mm | None (alloy) |
| C1 | Carbon Steel traced+insulated | Molten sulfur | 6 | 160 | ASME B31.3 | 1.5 mm | Heat trace + insulation |
| C2 | Carbon Steel | Cooling water | 8 | 60 | ASME B31.3 | 2.0 mm | Internal epoxy lining |
| D1 | HDPE-lined carbon steel | Dilution water | 10 | 60 | ASME B31.3 | 0 mm + HDPE | 2 mm HDPE liner |

# PROCESS LINE REGISTER — UNIT 100 TO 200 (SULFUR AND FURNACE)

| Line Number | Description | From | To | Fluid | Phase | Size (in) | Pipe Class | Flow Normal (kg/h) | Flow Surge (kg/h) | Velocity @ Surge (m/s) | Max Allow. Velocity (m/s) | Design P (barg) | Design T (°C) | Insulation | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100-MLT-4-C1-001 | Molten Sulfur Storage to Filter | TK-101 | F-101 | Molten S | Liq. | 4 | C1 | 1,363 | 1,499 | 0.9 | 1.5 (erosion limit) | 6 | 160 | Traced+insul. | PASS |
| 100-MLT-3-C1-002 | Molten Sulfur Filter to Furnace Gun | F-101 | BU-201 | Molten S | Liq. | 3 | C1 | 1,363 | 1,499 | 1.2 | 1.5 | 6 | 160 | Traced+insul. | PASS |
| 100-MLT-1-C1-003 | Sulfur Pump Recirculation Line | P-101 | TK-101 | Molten S | Liq. | 1 | C1 | 200 | 220 | 1.0 | 1.5 | 6 | 160 | Traced+insul. | PASS |
| 200-SGTP-48-B1-001 | Furnace Exit Gas to WHB Inlet | BU-201 | WHB-201 | SO₂ gas | Gas | 48 | B1 | 15,277 | 16,805 | 8.2 | 10.0 (erosion) | 0.15 | 1,050 | Refractory | PASS |
| 200-SGTP-36-B1-002 | WHB Outlet Gas to HX Network | WHB-201 | HX-401 | SO₂ gas | Gas | 36 | B1 | 15,277 | 16,805 | 5.6 | 10.0 | 0.12 | 420 | External insul. | PASS |
| 200-STM-8-A1-001 | WHB HP Steam to Export Header | WHB-201 | PCV-202 | Steam | Vapour | 8 | A1 | 7,500 | 8,250 | 18.5 | 25.0 | 42 | 415 | Insulated | PASS |
| 200-BFW-4-A1-001 | BFW to WHB Steam Drum | P-202 | WHB-201 | BFW | Liq. | 4 | A1 | 7,700 | 8,470 | 1.8 | 3.0 | 45 | 120 | Insulated | PASS |
| 200-AIR-20-A2-001 | Process Air — Blower to Furnace | K-301 | BU-201 | Dry Air | Gas | 20 | A2 | 11,480 | 12,628 | 11.2 | 15.0 | 0.5 | 60 | None | PASS |

# PROCESS LINE REGISTER — UNIT 300 (DRYING AND AIR BLOWER)

| Line Number | Description | From | To | Size (in) | Pipe Class | Flow Surge (kg/h or Nm³/h) | Velocity @ Surge (m/s) | Vel. Limit (m/s) | Design P (barg) | Design T (°C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 300-AIR-30-A2-001 | Atmosphere Air Inlet to DT | Atmosphere | T-301 | 30 | A2 | 9,760 Nm³/h | 1.4 | 2.5 | 0.1 | 45 |
| 300-AIR-20-A2-002 | Dried Air DT Outlet to Blower | T-301 | K-301 | 20 | A2 | 9,760 Nm³/h | 2.2 | 3.5 | 0.2 | 45 |
| 300-H2S4-4-B2-001 | DT Circulating Acid (supply) | P-301 | T-301 | 4 | B2 | 72 m³/h | 1.3 | 2.0 | 2.0 | 80 |
| 300-H2S4-4-B2-002 | DT Acid to Acid Cooler | T-301 | E-301 | 4 | B2 | 72 m³/h | 1.3 | 2.0 | 2.0 | 90 |
| 300-H2S4-4-B2-003 | DT Acid Cooler Return | E-301 | P-301 | 4 | B2 | 72 m³/h | 1.3 | 2.0 | 2.0 | 80 |
| 300-CW-6-C2-001 | CW to DT Acid Cooler | CW HDR | E-301 | 6 | C2 | 65 m³/h | 1.2 | 3.0 | 4.0 | 45 |
| 300-H2S4-2-B2-004 | Acid Make-up to DT Circuit | TK-701 | T-301 | 2 | B2 | 500 kg/h max | 0.6 | 1.5 | 4.0 | 80 |

# PROCESS LINE REGISTER — UNIT 400 (CONVERTER GAS LINES)

| Line Number | Description | Fluid | Size (in) | Pipe Class | Flow Surge (Nm³/h) | Velocity @ Surge (m/s) | Vel. Limit | Design P (barg) | Design T (°C) | Material Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 400-SO2G-36-B1-001 | Converter Pass 1 Inlet | SO₂/SO₃ gas | 36 | B1 | 9,760 | 5.4 | 10.0 | 0.15 | 425 | 316SS; refractory expansion joint |
| 400-SO2G-30-B1-002 | Converter Pass 1 Outlet (hot) | SO₂/SO₃ gas | 30 | B1 | 9,760 | 7.2 | 10.0 | 0.15 | 621 | FAIL: Design T in pipe class B1 set at 650°C — surge temp 621°C is within class but close (95%). Review thermal expansion at surge. |
| 400-SO2G-30-B1-003 | Interpass HX 1-2 Shell Side | SO₂/SO₃ gas | 30 | B1 | 9,760 | 5.8 | 10.0 | 0.12 | 520 | 316SS |
| 400-SO2G-30-B1-004 | Converter Pass 2 Inlet | SO₂/SO₃ gas | 30 | B1 | 9,760 | 5.8 | 10.0 | 0.12 | 442 | 316SS |
| 400-SO2G-24-B1-005 | Converter Pass 2 Outlet | SO₂/SO₃ gas | 24 | B1 | 9,760 | 7.1 | 10.0 | 0.12 | 526 | 316SS |
| 400-SO2G-24-B1-006 | Converter Pass 3 Inlet | SO₂/SO₃ gas | 24 | B1 | 9,760 | 7.1 | 10.0 | 0.12 | 432 | 316SS |
| 400-SO2G-20-B1-007 | Converter Pass 3 to IPAT | SO₂/SO₃ gas | 20 | B1 | 9,760 | 7.5 | 10.0 | 0.12 | 484 | 316SS |
| 400-SO2G-20-B1-008 | IPAT Outlet to Pass 4 | SO₂/SO₃ gas | 20 | B1 | 9,760 | 5.2 | 10.0 | 0.12 | 420 | 316SS |
| 400-SO2G-18-B1-009 | Converter Pass 4 Inlet | SO₂/SO₃ gas | 18 | B1 | 9,760 | 5.8 | 10.0 | 0.12 | 416 | 316SS |
| 400-SO2G-18-B1-010 | Converter Pass 4 to FAT | SO₂/SO₃ gas | 18 | B1 | 9,760 | 5.8 | 10.0 | 0.12 | 432 | 316SS |

# PROCESS LINE REGISTER — UNIT 500 (ABSORPTION) AND UNIT 700 (PRODUCT)

| Line Number | Description | Fluid | Size (in) | Pipe Class | Flow Surge | Velocity @ Surge | Design P | Design T (°C) | Lining |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 500-H2S4-8-B2-001 | IPAT Circulating Acid (circ.) | H₂SO₄ 98% | 8 | B2 | 132 m³/h | 1.8 m/s | 4.0 barg | 80 | Brick/rubber |
| 500-H2S4-6-B2-002 | IPAT Acid to Cooler | H₂SO₄ 98% | 6 | B2 | 132 m³/h | 2.5 m/s | 4.0 barg | 100 | Brick/rubber |
| 500-H2S4-6-B2-003 | IPAT Acid Cooler Return | H₂SO₄ 98% | 6 | B2 | 132 m³/h | 2.5 m/s | 4.0 barg | 75 | Brick/rubber |
| 500-H2O-1-D1-001 | Dilution Water to IPAT | DM Water | 1 | D1 | 242 L/h | 0.4 m/s | 10 barg | 40 | HDPE-lined |
| 500-H2S4-8-B2-004 | FAT Circulating Acid | H₂SO₄ 98% | 8 | B2 | 110 m³/h | 1.5 m/s | 4.0 barg | 80 | Brick/rubber |
| 500-H2S4-6-B2-005 | FAT Acid to Cooler | H₂SO₄ 98% | 6 | B2 | 110 m³/h | 2.1 m/s | 4.0 barg | 100 | Brick/rubber |
| 500-SO2G-16-B1-011 | FAT Outlet (Tail Gas) to Stack | SO₂/N₂ | 16 | B1 | 9,760 Nm³/h | 9.8 m/s | 0.05 barg | 85 | HDPE duct liner |
| 500-H2S4-6-B3-001 | Product Transfer to Storage | H₂SO₄ 98% | 6 | B3 | 4,583 kg/h | 0.8 m/s | 4.0 barg | 40 | Saramet |
| 700-H2S4-8-B2-001 | Product Tank to Dispatch Pump | H₂SO₄ 98% | 8 | B2 | 5,417 kg/h | 0.6 m/s | 2.0 barg | 40 | Brick/rubber |
| 700-H2S4-4-B3-001 | Dispatch Pump to Loading Arm | H₂SO₄ 98% | 4 | B3 | 5,417 kg/h | 1.8 m/s | 10 barg | 40 | Saramet |

# UTILITY LINE REGISTER (SELECTED)

| Line Number | Description | Size (in) | Pipe Class | Design P (barg) | Design T (°C) | Max Op P (barg) | Utilisation (P) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UTL-STM-HP-8-A1-001 | HP Steam Export Header 40 barg | 8 | A1 | 50 | 430 | 42 | 84% |
| UTL-STM-MP-4-A1-002 | MP Steam Utility Header 10 barg | 4 | A1 | 15 | 200 | 10 | 66% |
| UTL-CW-12-C2-001 | CW Supply Main Header | 12 | C2 | 8 | 50 | 4.5 | 56% |
| UTL-CW-12-C2-002 | CW Return Main Header | 12 | C2 | 8 | 50 | 3.5 | 44% |
| UTL-IA-2-A2-001 | Instrument Air Ring Main | 2 | A2 | 9 | 50 | 7.0 | 78% |
| UTL-N2-2-A2-001 | Nitrogen Supply Header | 2 | A2 | 9 | 50 | 7.0 | 78% |
| UTL-BFW-4-A1-001 | BFW Supply to WHB | 4 | A1 | 50 | 130 | 44 | 88% |
| UTL-DRN-ACID-4-B2-001 | Acid Drain/Sump Header | 4 | B2 | atm | 60 | atm | — |